State Water Resources Control Board

Division of Drinking Water                                                                             	October 2026

# Preparing Your CaliforniA Drinking Water Consumer Confidence Report (CCR)

Reference Manual for Water Systems

October 2026 Update

## Disclaimer

This document provides information to water systems on the State Water Resources Control Board (State Water Board), Division of Drinking Water’s current interpretation of the California Consumer Confidence Report (CCR) regulations. This document is not a substitute for regulations; nor is it a regulation itself. Thus, it does not impose legally-binding requirements on the State Water Board or water systems and may not apply to a particular situation based upon its circumstances. This document does not confer legal rights or impose legal obligations upon any member of the public. While the State Water Board has made every effort to ensure the accuracy of the discussion in this document, the statutes, regulations, or other legally binding requirements determine the obligations of the regulated community. In the event of a conflict between the discussion in this document and any statute or regulation, this document would not be controlling.

The general description provided here may not apply to a particular situation based on the circumstances. Interested parties are free to raise questions and objections about the substance of this reference manual and the appropriateness of its application to a particular situation. State decision-makers retain the discretion to adopt approaches on a case-by-case basis that differ from this reference manual where appropriate. The State Water Board may change this reference manual in the future.

NOTE: Subsequent to the 2026 CCR, there may be changes in required content in terms of newly adopted Maximum Contaminant Levels, Public Health Goals, and other revised regulatory requirements. These will be posted on the State Water Board’s website at https://www.waterboards.ca.gov/drinking_water/certlic/drinkingwater/CCR.html, but the reference manual itself may not be revised accordingly, most specifically, the appendices. Therefore, the water system should be aware that this reference manual provides the details necessary to prepare only the CCR due July 2027.

## Table of Contents

Disclaimer	1

Table of Contents	2

Introduction	4

What is a consumer confidence report (CCR)?	4

Who must prepare a CCR?	5

When must a water system distribute its CCR?	5

What content is required in the CCR?	6

CHECKLIST FOR COMPLETED CCR - Basic CCR Requirements	8

CHECKLIST GUIDANCE FOR COMPLETED CCR – Basic CCR Requirements	10

Item 0: Summary of Report Contents	10

Item 1: Water System Information	11

Item 2: Source(s) of Water	13

Item 3: Definitions	14

Item 4: Reporting Levels of Detected Contaminants	15

Item 5: Instructions for Specific Chemicals	26

Item 6: Compliance with Other Drinking Water Regulations	31

Item 7: Variances and Exemptions	36

Item 8: Educational Information	36

Item 9: Six-Month Update	39

What should the CCR look like?	40

How must the CCR be distributed?	40

General	40

Consumers not receiving water bills	41

State and local agencies and media outlets	42

Electronic delivery	42

How long must the CCR be kept?	42

APPENDIX A: Regulated Contaminants with Primary Drinking Water Standards	43

Microbiological Contaminants	43

Radioactive Contaminants	46

Inorganic Contaminants	48

Synthetic Organic Contaminants including Pesticides and Herbicides	54

Volatile Organic Contaminants	60

Disinfection Byproducts, Disinfectant Residuals, and Disinfection Byproduct Precursors	65

APPENDIX B: Regulated Contaminants with Secondary Drinking Water Standards	67

APPENDIX C: Monitored Contaminants with No MCLs	68

Background	68

Federal UCMR 1 (2001 – 2003 Monitoring)	68

Federal UCMR 2 (2008 – 2010 Monitoring)	69

Federal UCMR 3 (2013 – 2015 Monitoring)	70

Federal UCMR 4 (2018 – 2020 Monitoring)	71

Federal UCMR 5 (2023 – 2025 Monitoring)	72

Reporting	74

APPENDIX D: State Contaminants with Notification Levels	75

APPENDIX E: California’s Detection Limits for Purposes of Reporting (DLRs)	78

APPENDIX F: Reporting Monitoring Data	79

Example: Reporting for one sampling site and one sampling date	79

Example: Reporting for one sampling site and multiple sampling dates	79

Example: Reporting for multiple sampling sites and one sampling date	80

Example: Reporting for multiple sampling sites and multiple sampling dates (compliance: four quarter or six-month average)	82

Example: Reporting of multiple sampling sites and multiple sampling dates (compliance: RAA on individual source basis)	82

Example: Reporting of multiple sampling sites and multiple sampling dates (compliance: LRAA – TTHM and HAA5)	83

For a System with No LRAA Exceedance:	83

For a System with an LRAA Exceedance:	84

Example: Reporting of turbidity results	85

For conventional or direct filtration:	85

For diatomaceous earth filtration:	86

For slow sand filtration:	86

For alternative filtration technology:	86

Example: Reporting of lead results	86

Example: Reporting of fluoride results	88

Example: Reporting of radioactivity results	89

Example: Reporting of sodium and hardness results	89

Example: Reporting of Stage 1 D/DBPR TOC treatment technique violations (surface water treatment plants with conventional treatment or precipitative softening)	89

Example: Reporting of LT2ESWTR treatment technique violations	90

Example: Reporting of GWR special notice for fecal indicator-positive groundwater source sample	91

Example Data – E. coli	91

Example Data – Enterococci or Coliphage	92

Example: Reporting of GWR treatment technique violations for failure to take corrective action for fecal indicator-positive groundwater source sample	93

APPENDIX G: CCR Certification Form (Suggested Format)	95

APPENDIX H: Translations of “Note of Importance” for CCR	97

APPENDIX I: Source Water Protection and Water Conservation Tips for Consumers	101

APPENDIX J: Acronyms, Abbreviations, and Initials	103

## Introduction

This reference manual is intended to help water systems prepare their annual Consumer Confidence Reports (CCR). It explains the requirements for report content, format, and distribution required for conformance with the California Code of Regulations (Cal. Code Regs.) [Title 22, Chapter 15, Article 20] and law [California Health and Safety Code, HSC, § 116470]. The state regulations took effect on May 26, 2001, and were subsequently amended on September 1, 2006, with the adoption of the Public Notification regulations. Additional information on drinking water related laws and regulations is available on the State Water Board’s website (https://www.waterboards.ca.gov/drinking_water/certlic/drinkingwater/Lawbook.html).

As the system operator/manager, you are a guardian of the quality of your drinking water supply and of the public health in your community. It is important to communicate to your customers, and your customers have the right to know, the source of their water and what is in the water they drink. CCRs help consumers make informed choices that affect the health of themselves and their families. The reports also encourage consumers to consider and appreciate the challenges of delivering safe drinking water. Educated consumers are more likely to help protect their drinking water sources and to understand the true costs of safe drinking water.

Water systems, states, and the United States Environmental Protection Agency (U.S. EPA) work to educate consumers about the sources, quality, and delivery of their drinking water. Systems and states encourage consumers to participate in decision-making regarding source water assessment and protection programs and use of the Drinking Water State Revolving Fund (DWSRF), which provides funding for infrastructure upgrades and treatment improvements. Consumers who are familiar with the basic drinking water information in CCRs will be able to participate more effectively in these processes.

## What is a consumer confidence report (CCR)?

In 1996, Congress amended the Safe Drinking Water Act (SDWA), adding a requirement that water systems deliver to their customers a brief annual water quality report, similar to the Annual Water Quality Report (AWQR) that California water systems began distributing in 1990. However, CCR regulatory requirements are more specific and detailed in terms of content and format than those for the AWQR. A CCR summarizes information that your water system already collects to comply with regulations. The CCR regulation does not require you to engage in any new monitoring to complete your CCR.

The CCR includes information on source water, levels of any detected contaminants, and compliance with drinking water regulations (including monitoring requirements), along with some educational information. Most reports fit on a few sheets of paper. A report that contains too much information or is full of technical jargon can discourage consumers from learning about their drinking water. Beyond a mandatory requirement, a CCR is an opportunity to communicate the value of water (both as a product and as a service), promote wise use, build community trust and customer satisfaction, and encourage investment in resource protection and infrastructure.

## Who must prepare a CCR?

Every community water system (CWS) and every non-transient non-community water system (NTNCWS) must prepare and distribute a report.

A system may contract with a laboratory or other third-party to provide monitoring data analysis or CCR development assistance. If the system chooses to use a laboratory/third-party to assist with the development of the CCR, the system must work with the laboratory/third-party to make sure that all required elements are included in the CCR. Otherwise, a system may need to add the missing elements. Regardless of who prepares the CCR, the system is ultimately responsible for the content and must always distribute the CCR to its customer.

Wholesale systems (drinking water systems that sell water to one or more systems) are not responsible for creating a CCR for their consecutive systems (systems that purchase water from the wholesale system), nor are they responsible for providing data on contaminants that the consecutive system monitors (such as total coliforms, lead, or total trihalomethanes (TTHMs)). However, wholesale systems are responsible for providing the consecutive system with relevant source information and monitoring and compliance data so that the consecutive system can include this information in their CCR.

In some cases, a consecutive system will contract with the wholesale system to produce the report. There are several options in this relationship. If the consecutive system had no new data to add, it could simply send out the wholesale system’s CCR with a cover letter explaining their relationship. If the consecutive system did need to add data, it might choose to reprint the wholesaler system’s CCR with a new title/letterhead and the additional data (most consecutive systems will at least need total coliform data). Either of these options is acceptable. Regardless of who produces the CCR, the consecutive system is still responsible for ensuring that its customers receive a report containing all required content.

## When must a water system distribute its CCR?

You must distribute your CCR to consumers by July 1 of each year. If your system serves at least 10,000 people, you must also distribute your CCR a second time, by December 31 of each year. The CCR is based on data collected during, or prior to, the previous calendar year. For example, data collected between January 1 and December 31, 2026, must be reported in the 2026 CCR, which is due to customers by July 1, 2027, and/or December 31, 2027. If you monitor less frequently than annually, you will need to use your most recent data even though it is outside of the previous calendar year (further discussed in Checklist Item 4). If required to distribute the CCR twice each year, by December 31, some information from between January 1, 2027, and June 30, 2027, may be required in a 6-month update (further discussed in Checklist Item 9).

A new CWS or NTNCWS must deliver its first report by July 1 of the year after its first full calendar year in operation.

A wholesale system must provide the consecutive system with the previous calendar year’s monitoring data and other information by April 1 of each year unless the two systems make a different contractual agreement. If selling water to a system required to provide reports twice per year, the wholesale system must also provide any other information required in the 6-month update (Checklist Item 9) by October 1 of each year unless the two systems make a different contractual agreement. This gives the consecutive system enough time to prepare their CCR and 6-month update, if applicable, before the distribution deadlines.

## What content is required in the CCR?

This reference manual describes California’s requirements for a CCR and suggests (using the words “we encourage,” “should,” or “may”) other sections or explanations that will help your customers understand the report. Note that California requires more information and, in some cases, different information than the federal rule, so be sure to follow state regulations and this reference manual, not the federal rule or guidance. If you are familiar with the federal requirements, you should be aware of the following differences between the federal and state rules; the state regulations require:

- Both CWSs and NTNCWSs must distribute CCRs;

- Inclusion of public health goals (PHGs) in place of maximum contaminant level goals (MCLGs) for detected contaminants, unless no PHG has been adopted;

- Modified language for definitions;

- Additional definitions (PHG and primary drinking water standard, PDWS);

- Modified language for contaminant sources and health effects;

- Inclusion of secondary maximum contaminant levels (MCLs) for any detected contaminants along with any detected levels for sodium and hardness;

- Use of State MCLs only (U.S. EPA MCLs are not required);

- In addition to information on how to obtain a copy of a completed source water assessment, both the completion date (or when last updated) and a vulnerability summary written by the party conducting the assessment;

- A notice in Spanish informing people that the information therein is important; notices in other languages are required if specific regulatory criteria is met (see Cal. Code Regs., Title 22, § 64481(l)).

The State Water Board encourages you to tailor the content of your CCR to local conditions, as long as it meets the federal and state regulations. If you think that an added picture or graph would help your customers to understand your report, add it. If your customers would benefit from an explanation of your need for new treatment facilities, tell them. Provide information to your consumers in a way that they understand. For example, when discussing units of measure, explain it in terms that a consumer may understand, as shown in the table below. As long as any additional educational information is consistent with, and not detracting from, the purpose of the report, you may add it. For example, the CCR regulation does not require a title for your report. However, you should give your report a title to catch the customer’s attention. You may call the report a “Consumer Confidence Report,” a “Water Quality Report,” or choose another title.

| Units | Units | Equivalence |
| --- | --- | --- |
| mg/L = milligrams per liter | ppm = parts per million | 1 second in 11.5 days |
| µg/L = micrograms per liter | ppb = parts per billion | 1 second in nearly 32 years |
| ng/L = nanograms per liter | ppt = parts per trillion | 1 second in nearly 32,000 years |
| pg/L = picograms per liter | ppq = parts per quadrillion | 1 second in nearly 32,000,000 years |

Customers are most interested in a clear statement of whether or not their drinking water meets all standards. Although it is not required by the regulations, you will help your customers if you tell them whether their water met all drinking water standards. Be cautious in using the word “safe” since water that meets standards and is safe for most people might not be safe for infants, chemotherapy patients, or people with HIV/AIDS. Also, using the term “safe” if you have had an MCL or regulatory action level (AL) exceedance can be misleading to customers.

EXAMPLE – Last year, as in years past, your tap water met all U.S. EPA and State drinking water health standards. [INSERT WATER SYSTEM NAME] vigilantly safeguards its water supplies and once again, we are proud to report that our system has never violated a maximum contaminant level or any other water quality standard. [OR IF YOU HAD A VIOLATION, BEGIN WITH] Last year, we conducted more than [INSERT NUMBER OF TESTS] tests for over 80 contaminants. We only detected [INSERT CONTAMINANTS DETECTED] of these contaminants, and found only [INSERT NUMBER OF CONTAMINANTS DETECTED] at a level higher than the State allows. As we told you at the time, our water temporarily exceeded drinking water standards. For more information, see the paragraph marked Violation on the back.] This brochure is a snapshot of last year’s water quality. Included are details about where your water comes from, what it contains, and how it compares to State standards. We are committed to providing you with information because informed customers are our best allies.

Research conducted by the Water Research Foundation (formerly the American Water Works Association Research Foundation) described three important phases in facilitating customer understanding of the information in a CCR:

- Initial Sort: Customers are less likely to discard the CCR as “junk mail” if it looks professional, distinct, and prominently displays the utility’s name. However, glossy full-color reports are not necessary.

- Skimming: For the reader who chooses to skim the document, important and concise messages about water quality that are prominently displayed will attract attention. However, statements about the safety of water should not be over-stated, and specific warnings regarding health risks for sensitive sub-populations must be included. The use of color will draw attention and can be used to guide the reader through the CCR. Maps, simple tables, and photographs present information quickly and effectively.

- Reading: If the above challenges are addressed, a customer will hopefully choose to read the entire CCR. The document should not be designed to persuade the reader; it should inform the reader. A brief table of contents at the very beginning will help to guide the reader. Contaminant tables should be simple and should not require special instructions. The use of large fonts in an uncrowded format is desirable. Discussions regarding detected contaminants are helpful and should promote creditability.

Eight basic items must be included in all CCRs. These items are summarized in the Checklist for Completed CCR shown on the next page, and further discussed in detail below.

### CHECKLIST FOR COMPLETED CCR - Basic CCR Requirements

Item 0 – Summary of Report Contents

- A summary of violations, action level exceedances, significant deficiencies, E. coli detections, Level 1 and 2 assessments, and any related required information

- Contact information of owner, operator, or designee of the system

- Where consumers may obtain a translated copy of the report or get assistance in other languages, as required

- If using an electronic delivery method, prominently display directions to request a paper copy of the report

- If using the report to also meet public notification requirements, specify such and provide a brief description

- If part of a 6-month update, a brief description of the report and update, noting the availability of new information for the current year

- Standard language to encourage distribution

Item 1 – Water System Information

- Name and phone number of contact person

- Information on public participation opportunities

- Information in Spanish that report content is important or offer additional information

- Information for other non-English speaking populations, if applicable

Item 2 – Sources of Water

- Type, name, and location of water sources

- If source water assessment completed: completion date (or when last updated), availability and how to obtain it, and vulnerability assessment

Item 3 – Definitions (specific language)

- Contaminant

- MCL

- MCLG

- PHG

- PDWS

- Others as needed (MRDL, MRDLG, AL, TT, variances and exemptions, etc.)

Item 4 – Reported Levels of Detected Contaminants (in one or more tables)

- Summary of data on detected regulated and unregulated contaminants [both federal and state lists]

- MCL or MRDL expressed as a number equal to or greater than 1.0 and the PHG (or MCLG) or MRDLG in the same units

- TT or AL designation if there is no MCL or MRDL

- Compliance monitoring data in MCL/MRDL units for year of report, with detected level and range of sample results (see regulations and information in Appendix F)

- For turbidity: reporting differs (see regulations and information in Appendix F)

- For coliforms: reporting differs (see regulations and information in Appendix F)

- For lead/copper: 90th percentile value, number of sites sampled, number of sites exceeding ALs, and range of tap sampling results

- For unregulated contaminants: average and range of contaminant detections

- If monitoring less than once a year: date of most recent sample, result, and statement that data are from most recent sampling

- Known or likely source of each detected contaminant with an MCL/MRDL/TT/AL

- MCL/MRDL/TT/AL violations highlighted

- Definitions of all units used in the table

Item 5 – Instructions for Specific Chemicals

- If applicable, informational statement on arsenic

- If detected, monitoring results for chromium (hexavalent)

- If chromium (hexavalent) exceeds the MCL before the applicable compliance date, include required language with description of compliance actions

- If detected, warning for vulnerable populations about Cryptosporidium

- If applicable, informational statement on lead, including instructions to access the publicly accessible service line inventory

- If applicable, informational statement on nitrate

- If detected, explanation of radon and its presence in the finished water

- If detected, explanation of unregulated contaminants and their presence in drinking water

Item 6 – Compliance with Other Drinking Water Regulations

- Explanation of violation: length of violation, potential health effects (health effects language available for primary MCL, MRDL, TT and AL), and steps taken to correct the violation

- Special notices for groundwater users to meet the requirements of the Ground Water Rule (GWR)

- Language for Level 1 and Level 2 Assessment Requirements and TT Violations, Escherichia coli (E. coli) MCL Violation, and E. coli Detection

Item 7 – Variances and Exemptions

o Explanation of variance/exemption, if applicable

Item 8 – Required Educational Information (specific language)

- Explanation of contaminants and their presence in drinking water

- Explanation regarding contaminants that may reasonably be expected to be found in drinking water, including bottled water

- Information to customers that some people may be more vulnerable to contaminants in drinking water

Item 9 – Six-Month Update, if applicable

- A short description of the update and twice yearly delivery

- For any MCL, MRDL, or TT violations, the required contaminant section information and a readily understandable explanation of the violation

- For any other violation, a summary of the related information required in the report

- For lead action level exceedances occurring between January 1 and June 30 of the current year (2027), all required related information

- For UCMR results collected during the reporting year but not included in the reports distributed by July 1, the average and range of results for detected contaminants

### CHECKLIST GUIDANCE FOR COMPLETED CCR – Basic CCR Requirements

#### Item 0: Summary of Report Contents

You must prominently display a summary at the beginning of the report, including a brief description of the nature of the report.

The summary should include:

- A summary of violations and compliance information included in the report required by any:

  - Violations,

  - Lead action level exceedances,

  - Significant deficiencies (per the Ground Water Rule),

  - E. coli detections, and

  - Level 1 and Level 2 assessments.

- Contact information of the owner, operator, or designee of the system as a source of additional information concerning the report.

- Information where consumers may obtain a translated copy of the report or get assistance in:

  - Spanish, and

  - Any other non-English language for which the speaking group exceeds 1,000 residents or 10% of residents.

- If using an electronic delivery method, prominently display directions for consumers to request a paper copy of the report.

- If using the report to also meet public notification requirements:

  - Specify that the report is also serving to provide public notification of one or more violations or situations,

  - Provide a brief statement about the nature of the notice(s), and

  - Include a brief description of how to locate the notice(s) in the report.

- If part of a 6-month update with the second report, a brief description of the report and update, noting the availability of new information for the current year (between January and June).

- Standard language to encourage distribution of the report to all persons served: “Please share this information with anyone who drinks this water (or their guardians), especially those who may not have received this report directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this report in a public place or distributing copies by hand, mail, email, or another method.”

The summary must be written in plain language and may use infographics.

#### Item 1: Water System Information

Identify the name of your water system, and provide the following information about it:

- The name and telephone number of a person at the water system who can answer questions about the report;

- Information about how to access an associated website, if the system uses such a website to share additional information;

- A list of known opportunities for public participation in decisions that affect drinking water quality (e.g., time and place of regularly-scheduled water board or city/county council meetings). If you do not have regularly-scheduled meetings, tell customers how to get information when meetings are announced.

Consistent with the Cal. Code Regs., Title 22, § 64481(l), all water systems in California must contain information in Spanish expressing the importance of the report or offer additional information (e.g., a telephone number or address where Spanish-speaking residents may contact the system to obtain a translated copy of the report in Spanish or assistance in Spanish). In addition, for each non-English speaking group other than Spanish-speaking that exceeds 1,000 residents or 10 percent of the residents in the community, the report must:

- Include information in the appropriate language(s) expressing the importance of the report, and

- Either:

  - Contain information where such consumers may obtain a translated copy of the report or assistance in the appropriate language(s), or

  - The report must be in the appropriate language(s).

Systems that serve 100,000 or more people must develop a plan for providing assistance to consumers with limited English proficiency. The plan should be based on an evaluation of languages spoken by persons with limited English proficiency served by the water system. The plan should include the system’s anticipated approach to address translation needs. The first plan must be provided to the State Water Board with the first report in 2027. Plans must be evaluated annually and updated as necessary and reported with the CCR certification.

To help determine what languages are needed, water systems can get information on non-English speaking populations from census data (https://www.census.gov/), county offices, and local agencies supporting different ethnic groups. For additional languages spoken by more than 1,000 residents or 10 percent of the residents, the suggested statement shown below can be included in the CCR to inform customers of the importance of the CCR, and where they can find assistance. Appendix H presents translations of the suggested statement in 18 of the languages most spoken in California.

EXAMPLE – This report contains important information about your drinking water. Please contact [INSERT WATER SYSTEM NAME] at [INSERT WATER SYSTEM ADDRESS OR PHONE NUMBER] for assistance in [INSERT LANGUAGE]. - Spanish: Este informe contiene información muy importante sobre su agua para beber. Favor de comunicarse [INSERT WATER SYSTEM NAME] a [INSERT WATER SYSTEM ADDRESS OR PHONE NUMBER] para asistirlo en español. - Tagalog: Ang pag-uulat na ito ay naglalaman ng mahalagang impormasyon tungkol sa inyong inuming tubig. Mangyaring makipag-ugnayan sa [INSERT WATER SYSTEM NAME AND ADDRESS] o tumawag sa [INSERT WATER SYSTEM NAME] para matulungan sa wikang Tagalog. - See Appendix H for additional translations of the above sentences.

#### Item 2: Source(s) of Water

Describe your water type (e.g., groundwater, surface water, or a blend), the commonly-used name(s) (if such a name exists), and the general locations of your water source(s) (e.g., Well 1 located in our service area; East Well from the name-of-aquifer; or South Spring located in name-of-foothill, mountain, or watershed area). We encourage you to provide a simple map of your system’s sources without a detailed description of their location for security reasons. Note that water systems currently have the flexibility to address security concerns. Listing the water body where the intake is located for a surface water source and the name of the principal aquifer for a groundwater source would be appropriate. Treatment plant location is not required.

For more complicated systems, explaining your various interconnections and back-up sources may be difficult, but it is important that consumers understand that the source of their water may vary during the year. Remember to include in your table of detected contaminants monitoring data for these “additional” sources if you use water from them. If your situation is complex, you may need to describe the types of sources and how they are used; work with someone from the State Water Board or Local Primacy Agency (LPA) to decide what information belongs in your report.

If a source water assessment has been completed, tell customers the date it was completed (or last updated), that it is available, where to get a copy, and provide a brief summary of your source water’s vulnerability to contamination based on the assessment findings.

In cases where the information is available, we encourage you to highlight potential significant source of contamination in the source water area. Including this information in the CCR is an opportunity for you to provide customers with an explanation for why a contaminant is present in the source water.

If the State Water Board or LPA conducted the assessment, it will provide the summary for you to include. If you conducted your own assessment, you may write the summary yourself. The following is an example provided by the Drinking Water Source Assessment and Protection Program.

EXAMPLE – An assessment of the drinking water source(s) for [INSERT WATER SYSTEM NAME] was completed in [INSERT MONTH AND YEAR]. The source(s) are considered most vulnerable to the following activities associated with contaminants detected in the water supply: [INSERT SOURCES]. In addition, the source is considered most vulnerable to these activities: [INSERT ACTIVITIES]. A copy of the complete assessment is available at [INSERT STATE WATER RESOURCES CONTROL BOARD, DIVISION OF DRINKING WATER DISTRICT OFFICE OR WATER SYSTEM ADDRESS]. You may request a summary of the assessment be sent to you by contacting [INSERT STATE WATER RESOURCES CONTROL BOARD, DIVISION OF DRINKING WATER DISTRICT ENGINEER OR WATER SYSTEM REPRESENTATIVE] at [INSERT PHONE NUMBER].

If you do not have information from the source water assessment, we encourage you to include any other information about potential sources of contamination that is readily available to you; e.g., information contained in a sanitary survey.

We also encourage you to use the CCR as a way to discuss appropriate source water protection actions that are in the planning stages or are already in place. This discussion is an ideal opportunity to invite public participation in locally based source water protection efforts as well. Systems may also wish to provide ways they can protect the source water. Remember, this is your opportunity to educate your customers about the impacts they and others have on the quality of source water.

#### Item 3: Definitions

Every CCR must include definitions of key terms that consumers will need to understand the contaminant data. You must use the definitions listed below.

- Contaminant: Any physical, chemical, biological, or radiological substance or matter in water.

- Maximum Contaminant Level (MCL): The highest level of a contaminant that is allowed in drinking water. Primary MCLs are set as close to the PHGs (or MCLGs) as is economically and technologically feasible. Secondary MCLs are set to protect the odor, taste, and appearance of drinking water.

- Maximum Contaminant Level Goal (MCLG): The level of a contaminant in drinking water below which there is no known or expected risk to health. MCLGs are set by the U.S. Environmental Protection Agency.

- Public Health Goal (PHG): The level of a contaminant in drinking water below which there is no known or expected risk to health. PHGs are set by the California Environmental Protection Agency.

- Primary Drinking Water Standard (PDWS): MCLs, MRDLs and treatment techniques (TTs) for contaminants that affect health, along with their monitoring and reporting requirements.

Include the following definitions if the report contains data regulated using any of the following terms (i.e., chemical disinfectants, lead, copper, turbidity):

- Maximum Residual Disinfectant Level (MRDL): The highest level of a disinfectant allowed in drinking water. There is convincing evidence that addition of a disinfectant is necessary for control of microbial contaminants.

- Maximum Residual Disinfectant Level Goal (MRDLG): The level of a drinking water disinfectant below which there is no known or expected risk to health. MRDLGs do not reflect the benefits of the use of disinfectants to control microbial contaminants.

- Regulatory Action Level (AL): The concentration of a contaminant which, if exceeded, triggers treatment or other requirements that a water system must follow.

- Treatment Technique (TT): A required process intended to reduce the level of a contaminant in drinking water.

Include the following definition if your water system operated under a variance or exemption during the calendar year that the report describes:

- Variances and Exemptions: State Water Board permission to exceed an MCL or not comply with a TT under certain conditions.

Include the following definitions if your report contains information regarding a Level 1 or Level 2 assessment required under the State Revised Total Coliform Rule:

- Level 1 Assessment: A Level 1 assessment is a study of the water system to identify potential problems and determine (if possible) why total coliform bacteria have been found in our water system.

- Level 2 Assessment: A Level 2 assessment is a very detailed study of the water system to identify potential problems and determine (if possible) why an E. coli MCL violation has occurred and/or why total coliform bacteria have been found in our water system on multiple occasions.

Include the following definitions if your report contains any of the following terms:

- Herbicide: Any chemical(s) used to control undesirable vegetation.

- Pesticide: Generally, any substance or mixture of substances intended for preventing, destroying, repelling, or mitigating any pest.

#### Item 4: Reporting Levels of Detected Contaminants

An essential part of the CCR is the table that shows the highest level of each detected contaminant (this is usually the value you report to the State Water Board to determine compliance) and range of levels of that contaminant you found during the CCR calendar year (assuming more than one sample was collected). When treatment is provided and monitoring of the treated water is performed, the available treated water quality data may be used in lieu of untreated (raw) source water quality data (see page 18, Checklist Item 5.c). See Appendix F for how to interpret monitoring data and determine the levels to enter in the tables.

A detected contaminant is any contaminant detected at or above its detection level for purposes of reporting (DLR) (see Appendix E). Do not include contaminants in the table that are not detected, or are detected below the DLR. If you sometimes distribute water from emergency or back-up sources, you need to include monitoring results from these sources. Monitoring results must be reported for all sources used to supply water to customers, no matter how infrequently the source was used. Results for sources that were not used to deliver water to customers do not need to be reported.

The main table of detected contaminants must contain only data about: (1) regulated contaminants [contaminants subject to an MCL, MRDL, AL, or TT], (2) unregulated contaminants for which U.S. EPA or the State Water Board requires monitoring under Title 40 of the Code of Federal Regulations (CFR) section 141.40 or HSC section 116375(b), respectively, and (3) sodium and hardness. See Checklist Item 5 for special instructions about Cryptosporidium, radon, and other contaminants.

You may make several tables to separate regulated contaminants from unregulated contaminants. You may want to organize your table(s) by contaminant type (e.g., microbial, inorganic) or sampling site (e.g., treatment plant, distribution system). Report any additional monitoring data in another section of the CCR, separated from the regulated contaminant data (e.g., data specified in the regulations). If you want to list all the contaminants that you monitored but did not detect, you must do so outside of the table(s) of detected contaminants.

To ensure that consumers can easily compare detected contaminant levels to their MCLs, your table must display the MCL for each contaminant in units that express it as a number equal to or greater than 1.0. Appendix A shows the conversion factor for each contaminant. For contaminants with primary MCLs, report the PHG (use the MCLG if no PHG has been set) and level of the detected contaminant in the same units as the MCL. For example, atrazine is usually reported in mg/L. It is easier for customers to see that your water contains atrazine at a level two times lower than the MCL if you report the MCL as 1 µg/L and the detected level as 0.5 µg/L than if you were to report the MCL as 0.001 mg/L and the detected level as 0.0005 mg/L. In this case, you convert by multiplying the detected level and MCL by 1,000. When you round results to determine compliance, round before multiplying the results by these factors. If the MCL units for a contaminant are already expressed in a number greater than 1.0 (e.g., the MCL for nitrate is 10 mg/L as nitrogen), there is no conversion factor to apply to the MCL or detected contaminant level. For monitoring of disinfectant residuals (i.e., chlorine, chloramines, and chlorine dioxide), as required under the Stage 1 Disinfectants and Disinfection Byproducts Rule, use the same approach for the MRDL and MRDLG.

The CCR includes data from monitoring completed during the previous calendar year. For example, data collected between January 1 and December 31, 2026, must be reported in the 2026 CCR, which is due to customers by July 1, 2027. However, if you have monitoring waivers, or for another reason monitor less than once per year, use your most recent data even though it is outside of the calendar year. For example, if you monitor once every three years for lindane and detect lindane in a sample, report that detected level each of the three years until you take a new sample.

If the report contains detection data that are not from the calendar year indicated, the table must show the date of monitoring and the report must contain a brief statement explaining that the data presented are from the most recent monitoring done in compliance with regulations. The following shows an example of explanation that can be used for this purpose.

EXAMPLE – The State allows us to monitor for some contaminants less than once per year because the concentrations of these contaminants do not change frequently. Some of our data, though representative, are more than one year old.

You do not need to report monitoring results that are more than nine years old.

##### The Table Must Contain the Following Information for each Detected Contaminant

1)	If the contaminant is regulated by a maximum contaminant level (MCL) or maximum residual disinfectant level (MRDL) (see Appendices A and B):

a)	The MCL or MRDL, expressed as a number equal to or greater than 1.0;

b)	The PHG (or MCLG, if no PHG has been set) or MRDLG, expressed in the same units as the MCL or MRDL;

Note: Secondary MCLs do not have PHGs or MCLGs because secondary MCLs are set to protect the aesthetics of water and PHGs and MCLGs are based on health concerns.

c)	The level of that contaminant expressed in the same units as the MCL and PHG (or MCLG, if no PHG has been set) or MRDL and MRDLG.

2)	If the contaminant is regulated by a treatment technique (TT) (see Appendix A):

a)	Put the letters “TT” in place of the MCL;

b)	Put “N/A” (not applicable) in place of the PHG or MCLG when no PHG or MCLG is listed in Appendix A.

3)	If the contaminant is regulated as an action level (AL) (see Appendix A):

a)	The AL expressed as a number equal to or greater than 1.0;

b)	The PHG (or MCLG, if no PHG has been set) in the same units as the AL.

4)	If the contaminant is unregulated (i.e., monitoring was required under the federal or state Unregulated Contaminant Monitoring Rule [UCMR]) (see Appendix C):

a)	The average level of that contaminant and the range of detections.

5) The level of the contaminant must be represented as follows (see Appendix F for examples):

a)	If compliance with an MCL is determined by a running annual average (RAA) of all the samples taken from a sampling point (e.g., chemical contaminants), include the highest RAA (as reported to the State Water Board for compliance purposes), and the range of sample results. If sampling points are summarized together, include the highest RAA of any of the sampling points and the range of sample results from all sampling points.

b)  If compliance with an MCL is determined any way other than RAA, report the results as follows:

- For a single sampling site, or multiple sampling sites for which data are being individually listed in the CCR, include the highest sample result, the average of sample results, and the range of sample results. If only one sample was collected at a site, the average and range are not required.

- For multiple sampling sites for which data are being summarized together in the CCR:

  - If each site has been sampled only once, include the highest sample result at any site, as well as the average and the range of sample results from all sites.

  - If at least one site has been sampled more than once, include the highest sample result at any site, the average of each individual sampling site, and the range of sample results from all sites.

  - If waters are entering the distribution system at the same point, a flow-weighted average may be reported.

c)	If compliance is determined by monitoring after treatment is installed to remove a contaminant, include the average level detected in the water entering the distribution system, and the range of sample results.

d)	If a compliance determination was made in the year for which sample results are being reported and the determination was based on an average of results from both the previous and reporting years, include the highest compliance determination average and the range; where the range is based only on results from the year for which data are being reported.

e) When rounding of results is used to determine compliance with the MCLs (as allowed by regulations), rounding should be done prior to multiplying the results by the factor listed in Appendix A.

f) For total trihalomethanes (TTHM) and haloacetic acids (HAA5):

- Compliance is determined based on a locational running annual average (LRAA). Include the highest LRAA for TTHM and HAA5 and the range of individual samples results for all monitoring locations. If more than one monitoring location exceeds the TTHM or HAA5 MCL, include the LRAA for all locations that exceed the MCL.

g)	For turbidity:

- When turbidity is reported pursuant to Cal. Code Regs., Title 22, § 64449 (i.e., secondary MCLs), include the highest average monthly value.

- When turbidity is reported pursuant to Cal. Code Regs., Title 22, § 64652.5 (i.e., regulated as a TT for systems that have met criteria for avoiding filtration), include the highest single measurement found in any month and an explanation of the reasons for measuring turbidity. The following shows an example of explanation that can be used for this purpose.

EXAMPLE – Turbidity is a measure of the cloudiness of the water. We monitor it because it is a good indicator of water quality. High turbidity can hinder the effectiveness of disinfectants.

- When turbidity is reported pursuant to Cal. Code Regs., Title 22, § 64653 (i.e., regulated as a TT for systems that filter and use turbidity as an indicator of filtration performance), include the highest single measurement, the lowest monthly percentage of samples meeting the turbidity limits specified in section 64653 for the relevant filtration technology, and an explanation of the reasons for measuring turbidity. The following shows an example of explanation.

EXAMPLE – Turbidity is a measure of the cloudiness of the water. We monitor it because it is a good indicator of the effectiveness of our filtration system.

h)	For lead and/or copper:

- Include:

  - The 90th percentile value from the most recent round(s) of sampling (if it is a number greater than zero).

  - The number of sites sampled.

  - The number of sites that exceeded the AL.

  - The range of tap sampling results. Do not report related water quality parameter data.

  - Notification that complete lead tap sampling data are available for review and information on how to access the data.

  - A statement that a service line inventory (including inventories where the publicly accessible inventory consists of a written statement that there are no lead, galvanized requiring replacement, or lead status unknown service lines, known lead connectors or connectors of unknown material) has been prepared and instructions to access the publicly accessible service line inventory, including the direct line to the inventory if it is available online.

  - For systems with lead, galvanized requiring replacements, or lead status unknown service lines in the system’s inventory, information on how to obtain a copy of the service line replacement plan or a direct link to the plan if it is available online.

  - A plainly worded explanation of the corrosion control efforts the system is taking in accordance with regulations, using the appropriate template below:

    - For systems with Optimal Corrosion Control Treatment designated by the State Water Board or U.S. EPA:

Corrosion of pipes, plumbing fittings, and fixtures may cause lead and copper to enter drinking water. To assess corrosion of lead and copper, [NAME OF SYSTEM] conducts tap sampling for lead and copper at selected sites [INSERT FREQUENCY AT WHICH SYSTEM CONDUCTS TAP SAMPLING]. [NAME OF SYSTEM] treats water using [IDENTIFY TREATMENT METHOD] to control corrosion, which was designated as the optimal corrosion control treatment by [the State Water Board or U.S. EPA, as applicable]. To ensure the treatment is operating effectively, [NAME OF SYSTEM] monitors water quality parameters set by the [the State or U.S. EPA, as applicable] [INSERT FREQUENCY AT WHICH SYSTEM CONDUCTS WATER QUALITY PARAMETER MONITORING]. If applicable, add: [NAME OF SYSTEM] is currently conducting a study of corrosion control to determine if any changes to treatment methods are needed to minimize the corrosivity of the water.

    - For systems without Optimal Corrosion Control Treatment designated by the State Water Board or U.S. EPA:

Corrosion of pipes, plumbing fittings and fixtures may cause metals, including lead and copper, to enter drinking water. To assess corrosion of lead and copper, [NAME OF SYSTEM] conducts tap sampling for lead and copper at selected sites [insert frequency at which system conducts tap sampling]. If applicable, add: [NAME OF SYSTEM] treats water using [IDENTIFY TREATMENT METHOD] to control corrosion. If applicable add: [NAME OF SYSTEM] is currently conducting a study of corrosion control to determine if any changes to treatment methods are needed to minimize the corrosivity of the water.

- A statement that the water system is required to sample for lead in schools and licensed child care facilities as requested by the facility and that directs the public to contact their school or child care facility for further information about potential sampling results.

- For systems that exceeded the lead action level, clearly identify the exceedance if any corrective action has been required during the monitoring period covered by the report. The report must include a clear and readily understandable explanation of the exceedance, the steps consumers can take to reduce their exposure to lead in drinking water, and a description of any corrective actions the system has or will take to address the exceedance.

i)	For E. coli under the state Revised Total Coliform Rule (RTCR):

- For E. coli, include the total number of positive samples collected that year.

j)	For fecal indicator-positive source samples under the Ground Water Rule (GWR):

- For E. coli, list the MCL and MCLG as zero;

- For enterococci or coliphage, list “TT” in the column for MCL and “N/A” in the column for MCLG;

- For E. coli, enterococci, or coliphage, include the total number of positive samples for the year and special notice language provided in the table or elsewhere in the CCR. Refer to Checklist Item 6 for more information on special notice language requirements for fecal indicator-positive groundwater source samples.

k)	For beta particles:

- If you detect beta particles in your water at or below 50 pCi/L, you should report the detected level in pCi/L. So that consumers may have a standard against which to compare that detected level, you should include “50 pCi/L*” in the MCL column (rather than the actual MCL of 4 mrem/year) and a footnote to the table that says “*The State Water Resources Control Board considers 50 pCi/L to be the level of concern for beta particles.”

- If you detected beta particles above 50 pCi/L, you must determine the actual radioactive constituents present in the water to calculate the dose exposure level in mrem/year, and must report the detected level and MCL as mrem/year.

- In addition to detected contaminants, the CCR regulations require that all TT violations be reported in a detected contaminant table(s). TT violations are listed below and are organized by rule (refer to Checklist Item 6 for specific information about failure to install adequate filtration or disinfection equipment or processes or a failure of those processes, violations associated with acrylamide and epichlorohydrin, and violations associated with the Lead and Copper Rule, LCR).

- Surface Water Treatment Rule (SWTR), Interim Enhanced Surface Water Treatment Rule (IESWTR), and Long Term 1 Enhanced Surface Water Treatment Rule (LT1ESWTR)

- Failure to install adequate filtration or disinfection equipment or processes;

- Failure of the filtration or disinfection equipment or process;

- Failure to meet inactivation requirements at the treatment plant (CT value);

- Failure to maintain at least 0.2 mg/L disinfection residual at the distribution system entry point for more than 4 hours;

- Failure to maintain a distribution system disinfectant residual;

- Failure to meet source water quality conditions (only filtration avoidance systems);

- Failure to meet watershed control program requirements (only filtration avoidance systems);

- Failure to have redundant components for disinfection or automatic shut-off of water delivered to the distribution system (only filtration avoidance systems).

- Filter Backwash Recycling Rule (FBRR)

- Failure to return recycle flows through the processes of the existing filtration system or to an alternate State Water Board-approved location (only conventional and direct filtration systems).

- Long-Term 2 Enhanced Surface Water Treatment Rule (LT2ESWTR)

- Failure to cover an uncovered finished water reservoir, provide treatment of the reservoir’s discharge (to achieve inactivation and/or removal of at least 4-log virus, 3-log Giardia lamblia, and 2-log Cryptosporidium using a protocol approved by the State Water Board), or be in compliance with a State Water Board-approved schedule to cover the reservoir(s) or treat the reservoir(s) discharge.

- Filtered systems

    - Failure to determine and report bin classification;

    - Failure to provide or install an additional level of treatment using a microbial toolbox option by the required date;

    - Failure to achieve required treatment credit to meet the bin classification requirements using a microbial toolbox option.

- Unfiltered systems

    - Failure to calculate and report mean Cryptosporidium level;

    - Failure to install a second disinfectant to treat for Cryptosporidium by required date;

    - Failure to achieve required inactivation level by required date;

    - Failure to maintain required inactivation level based on mean Cryptosporidium results.

- Ground Water Rule (GWR)

- Failure to maintain at least 4-log treatment of viruses for more than 4 hours for groundwater systems that are required to treat;

- Failure to take corrective action, if necessary based on a fecal indicator-positive sample, or be in compliance with a plan and schedule for a fecal indicator-positive groundwater source sample;

- Failure to take corrective action, if necessary based on a significant deficiency, or be in compliance with a plan and schedule for a significant deficiency.

- Lead and Copper Rule (LCR)

- Failure to meet corrosion control treatment, source water treatment, lead service line replacement, or public education requirements.

- Chemical

- Acrylamide – Exceedance of specified level (i.e., 0.05 percent monomer in polyacrylamide dosed at 1 mg/L, or equivalent);

- Epichlorohydrin – Exceedance of specified level (i.e., 0.01 percent residual at of epichlorohydrin dosed at 20 mg/L, or equivalent).

- State Revised Total Coliform Rule (RTCR)

- When a water system exceeds a coliform TT trigger specified in Cal. Code Regs., Title 22, § 64426.7, and then fails to conduct the required Level 1 or Level 2 Assessment or corrective actions within the timeframe specified in Cal. Code Regs., Title 22, § 64426.8.

- For a seasonal system, failure to complete a State Water Board-approved start-up procedure, to certify to the State Water Board the water system has complied with the State Water Board-approved start-up procedure, to submit to the State Water Board results of bacteriological and disinfectant residual monitoring, and to obtain written State Water Board approval prior to serving water to the public [Cal. Code Regs., Title 22, § 64426.9].

The State Water Board recommends that systems include TT violations listed in section (6) above in a table adjacent to the main detected contaminant table. The table must include an explanation of the violation, the length of the violation, any potential adverse health effects, and steps taken to correct the violation. The following is an example of a GWR TT violation.

| TT Violation | Explanation | Length | Steps Taken to Correct the Violation | Health Effects Language |
| --- | --- | --- | --- | --- |
| Failed to maintain 4-log treatment of viruses | On January 10, 2026, state inspection of our water system identified a malfunctioning chlorine pump. As a result, the water from one of our wells (Well 1) was not adequately disinfected for 2 weeks. | 2 weeks | As directed by the State Water Resources Control Board, we took immediate action to resolve this problem by repairing the malfunctioning chlorine pump. Regular testing since the pump was repaired has demonstrated that we are once again providing water that meets the state’s standards for disinfection to our customers. | Inadequately treated or inadequately protected water may contain disease-causing organisms. These organisms can cause symptoms such as diarrhea, nausea, cramps, and associated headaches. |

7)	The likely source(s) of that contaminant, to the best of the operator’s knowledge. Specific information regarding contaminants may be available in sanitary surveys and source water assessments, and should be used when available. If the source of the contamination is known, the CCR should identify a specific point source, such as “Al’s chicken houses” or the “Super-Shiny Paper Mill”. If you lack specific information on the likely source of a contaminant, include one or more of the typical sources listed in Appendix A (for chemicals with primary MCLs, MRDLs, TTs and ALs) and Appendix B (for chemicals with secondary MCLs) that is most applicable to your situation.

8)	For any contaminant detected in violation of a MCL, MRDL, or TT, or exceeding an AL, clearly highlight in the table the violation or exceedance. This indication could, for example, take the form of a different color type, a larger or bolder font, or a large star. Near, but not in, the table, include an explanation of the violation, including the length of the violation/exceedance, potential adverse health effects (from Appendix A for primary MCLs, MRDLs, TTs and ALs), and actions you took to address the violation/exceedance.

9)	If you have detected unregulated contaminants for which State Water Board or federal rules require(d) monitoring (e.g., federal UCMR, 40 CFR section 141.40, or HSC section 116375(b), except Cryptosporidium) include the average of all of the year’s monitoring results, the range of detections, and a brief explanation of the reasons for monitoring for unregulated contaminants. See Appendix C for lists of these contaminants.

EXAMPLE – Unregulated contaminant monitoring helps U.S. EPA and the State Water Resources Control Board to determine where certain contaminants occur and whether the contaminants should be regulated in the future.

We encourage you to include more information on the potential health effects of these contaminants if the results indicate a health concern. We consider any detection above a proposed MCL, California Notification Level (NL), or U.S. EPA health advisory level to indicate concern. You may call the Safe Drinking Water Hotline (1-800-426-4791) for more information on federal health advisories or find it at https://www.epa.gov/sdwa/drinking-water-health-advisories-has. California NLs are available on the State Water Board’s website (http://www.swrcb.ca.gov/drinking_water/certlic/drinkingwater/NotificationLevels.shtml). For these contaminants, the State Water Board recommends that the report contain an explanation of the significance of the results, noting the existence of the proposed MCL, California NL (see Appendix D for available optional health effects language), or U.S. EPA health advisory.

##### Multiple Distribution Systems

If your water system supplies water through two or more distribution systems that are not physically interconnected and that are fed by different raw water sources, you must issue a CCR that differentiates contaminant data for each service area and the report should identify each separate distribution system. For example, data tables could contain a separate column for each service area. Alternatively, systems could produce separate reports tailored to include data for each service area.

##### Including Tier 3 Public Notices in CCRs

If you are required to provide a Tier 3 public notice for a monitoring violation or other type of violation or situation, you may consider including the notice in your CCR. If you use the CCR for public notification, make sure you meet the content requirements under the Public Notification Rule. Also, remember that the timing and delivery requirements for CCRs differ from those for public notices. Be careful to adhere to the Public Notification Rule requirement that Tier 3 public notice be completed no later than 12 months from the date the violation or situation occurred. To minimize the timing conflict, you can publish the CCR early – as soon after the end of the calendar year as possible; or mail a separate public notice for the violations occurring in January through June of the current year in the same envelope as your CCR covering the previous calendar year’s violations.

#### Item 5: Instructions for Specific Chemicals

You must include in your CCR the relevant statements below in the specified situations. You may include additional information, either before or after the required statement.

##### Arsenic

Systems that detect arsenic above 5 µg/L (50 percent of the MCL), but below or equal to 10 µg/L (the MCL), must include the following statement:

Arsenic is known to cause cancer in humans. Arsenic also may cause other health effects such as skin damage and circulatory problems. [NAME OF UTILITY] meets the EPA arsenic drinking water standard, also known as a Maximum Contaminant Level (MCL). However, you should know that EPA’s MCL for arsenic balances the scientific community’s understanding of arsenic-related health effects and the cost of removing arsenic from drinking water. The highest concentration of arsenic found in [YEAR] was [INSERT MAX ARSENIC LEVEL] ppb.

##### Chromium (hexavalent)

If chromium (hexavalent) is detected, the CCR must include information pursuant to DLR reporting requirements listed in subsections (c) and (d) of section 64481.

If chromium (hexavalent) exceeds the MCL, and a CCR issued prior to an applicable MCL compliance date, shown below,

| System Size: Service Connections served on October 1, 2024 | MCL Compliance Date |
| --- | --- |
| 10,000 or greater | October 1, 2026 |
| 1,000 to 9,999 | October 1, 2027 |
| Fewer than 1,000 | October 1, 2028 |

the CCR must include this language:

Chromium (hexavalent) was detected at levels that exceed the chromium (hexavalent) MCL. While a water system of our size is not considered in violation of the chromium (hexavalent) MCL until after [INSERT APPLICABLE TABLE 64432-B COMPLIANCE DATE], we are working to address this exceedance and comply with the MCL. Specifically, we are [INSERT ACTIONS TAKEN AND PLANNED TO COMPLY WITH THE APPLICABLE COMPLIANCE DATE IN TABLE 64432-B].

##### Cryptosporidium

If your system has performed monitoring that indicates the presence of Cryptosporidium in either the source water or the finished water, you must include the following information separate from the detected contaminant table:

- A summary of the results of monitoring. You should identify if the data are for the source water or finished water. You may choose whether or not to report the actual analytical results as part of this summary.

- An explanation of the significance of the results. Tell customers if they need to be concerned by the information that the CCR provides. The following shows an example of explanation that can be used for this purpose.

EXAMPLE – Cryptosporidium is a microbial pathogen found in surface water throughout the U.S. Although filtration removes Cryptosporidium, the most commonly-used filtration methods cannot guarantee 100 percent removal. Our monitoring indicates the presence of these organisms in our source water and/or finished water. Current test methods do not allow us to determine if the organisms are dead or if they are capable of causing disease. Ingestion of Cryptosporidium may cause cryptosporidiosis, an abdominal infection. Symptoms of infection include nausea, diarrhea, and abdominal cramps. Most healthy individuals can overcome the disease within a few weeks. However, immuno-compromised people, infants and small children, and the elderly are at greater risk of developing life-threatening illness. We encourage immuno-compromised individuals to consult their doctor regarding appropriate precautions to take to avoid infection. Cryptosporidium must be ingested to cause disease, and it may be spread through means other than drinking water.

##### Lead

All water systems are required to comply with the state LCR. Water systems are also required to comply with the federal LCR, and its revisions and corrections. The revisions and corrections to the federal LCR include mandatory language requirements that have not yet been adopted by the State Water Board.

Consistent with 40 CFR sections 141.84(a)(7) and 40 CFR 141.153(h)(8)(ii), CCRs must include a statement that a service line inventory has been prepared and include instructions to access the publicly available service line inventory, including inventories consisting of only a statement that the distribution system has no lead. If the service line inventory is online, the CCR must include the direct link to the inventory.

Consistent with 40 CFR section 141.154(d)(1), every CCR must include the lead-specific language shown below. A water system may provide its own educational statement, but only after consulting with the State Water Board.

Lead can cause serious health effects in people of all ages, especially pregnant people, infants (both formula-fed and breastfed), and young children. Lead in drinking water is primarily from materials and parts used in service lines and in home plumbing. [INSERT NAME OF SYSTEM] is responsible for providing high quality drinking water and removing lead pipes but cannot control the variety of materials used in the plumbing in your home. Because lead levels may vary over time, lead exposure is possible even when your tap sampling results do not detect lead at one point in time. You can help protect yourself and your family by identifying and removing lead materials within your home plumbing and taking steps to reduce your family's risk. Using a filter, certified by an American National Standards Institute accredited certifier to reduce lead, is effective in reducing lead exposures. Follow the instructions provided with the filter to ensure the filter is used properly. Use only cold water for drinking, cooking, and making baby formula. Boiling water does not remove lead from water. Before using tap water for drinking, cooking, or making baby formula, flush your pipes for several minutes. You can do this by running your tap, taking a shower, doing laundry or a load of dishes. If you have a lead service line or galvanized requiring replacement service line, you may need to flush your pipes for a longer period. If you are concerned about lead in your water and wish to have your water tested, contact [INSERT NAME OF SYSTEM and CONTACT INFORMATION]. Information on lead in drinking water, testing methods, and steps you can take to minimize exposure is available at https://www.epa.gov/safewater/lead.

Consistent with the Cal. Code Regs., Title 22, § 64482(c), systems that detect lead above 15 ppb (the regulatory AL) in more than 5%, and up to and including 10%, of sites sampled (or if your system samples fewer than 20 sites and has even one sample above the AL) must also include the following statement:

Infants and young children are typically more vulnerable to lead in drinking water than the general population. It is possible that lead levels at your home may be higher than at other homes in the community as a result of materials used in your home’s plumbing. If you are concerned about elevated lead levels in your home’s water, you may wish to have your water tested and/or flush your tap for 30 seconds to 2 minutes before using tap water. Additional information is available from the U.S. EPA Safe Drinking Water Hotline (1-800-426-4791).

##### Nitrate

Systems that detect nitrate above 5 mg/L as nitrogen (50 percent of the MCL), but below 10 mg/L as nitrogen (the MCL), must include the following statement:

Even though [NAME OF UTILITY] meets the EPA nitrate drinking water standard, also known as a Maximum Contaminant Level (MCL), if you are caring for an infant and using tap water to prepare formula, you may want to use alternate sources of water or ask for advice from your health care provider. Nitrate levels above 10 mg/L pose a particularly high health concern for infants under 6 months of age and can interfere with the capacity of the infant’s blood to carry oxygen, resulting in a serious illness; symptoms include shortness of breath and blueness of the skin, known as “blue baby syndrome.” Nitrate levels above 10 mg/L may also affect the ability of the blood to carry oxygen in other individuals, such as pregnant women and those with certain specific enzyme deficiencies. If you are caring for an infant, or you are pregnant, you should ask advice from your health care provider.

If a utility cannot demonstrate to the State Water Board with at least five years of the most current monitoring data that its nitrate levels are stable, it must also add the following language to the preceding statement on nitrate:

Nitrate levels in drinking water may increase for short periods of time due to high levels of rainfall or agricultural activity, therefore we test for nitrate [INSERT APPLICABLE SAMPLING FREQUENCY]. The highest level for nitrate found during [YEAR] was [INSERT MAX NITRATE LEVEL] ppm.

##### Radon

If your system has performed monitoring that indicates the presence of radon in its finished water, you must include the results of the monitoring and an explanation of the significance of the results in your CCR. The following shows an example explanation that can be used for this purpose:

EXAMPLE – We constantly monitor the water supply for various contaminants. We have detected radon in the finished water supply in _____ out of _____ samples tested. There is no federal regulation for radon levels in drinking water. Exposure over a long period of time to air transmitting radon may cause adverse health effects.

The language below may be included if the level of information is helpful:

EXAMPLE – Radon is a radioactive gas that you cannot see, taste, or smell. It is found throughout the U.S. Radon can move up through the ground and into a home through cracks and holes in the foundation. Radon can build up to high levels in all types of homes. Radon can also get into indoor air when released from tap water from showering, washing dishes, and other household activities. Compared to radon entering the home through soil, radon entering the home through tap water will in most cases be a small source of radon in indoor air. Radon is a known human carcinogen. Breathing air containing radon can lead to lung cancer. Drinking water containing radon may also cause increased risk of stomach cancer. If you are concerned about radon in your home, test the air in your home. Testing is inexpensive and easy. You should pursue radon removal for your home if the level of radon in your air is 4 picocuries per liter of air (pCi/L) or higher. There are simple ways to fix a radon problem that are not too costly. For additional information, call your State radon program (1-800-745-7236), the U.S. EPA Safe Drinking Water Hotline (1-800-426-4791), or the National Safety Council Radon Hotline (1-800-767-7236).

##### Other contaminants

If your system has performed voluntary monitoring that indicates the presence of non-regulated contaminants in the finished water, we strongly encourage you to report any results that may indicate a health concern. Public knowledge of potential problems is in the interest of you and your customers. We consider any detection above a proposed MCL, California NL, or U.S. EPA health advisory level to indicate concern. Call the Safe Drinking Water Hotline (1-800-426-4791) or visit U.S. EPA’s website (https://www.epa.gov/sdwa/drinking-water-health-advisories-has) for more information on federal health advisories. California NLs are shown in Appendix D and available at http://www.swrcb.ca.gov/drinking_water/certlic/drinkingwater/NotificationLevels.shtml. For these contaminants, the State Water Board recommends that the CCR contain:

- The results of monitoring;

- An explanation of the significance of the results, noting the existence of the proposed MCL, California NL (see Appendix D for available optional health effects language), or U.S. EPA health advisory (https://www.epa.gov/sdwa/drinking-water-health-advisories-has).

#### Item 6: Compliance with Other Drinking Water Regulations

##### Other than the Ground Water Rule – Special Notice Requirements

If your water system violated one of the following requirements during the year covered by your CCR, the report must describe the violation(s). Just as you must explain the potential health effects of any MCL violation, you must provide a clear and readily understandable explanation of any other violation, potential adverse health effects (if any), and the steps the system has taken to correct the violation.

Treatment techniques (TT; must include length of violation)

- Filtration and disinfection requirements (SWTR, IESWTR, and LT1ESWTR). If you violated the TT discussed in Checklist Item 4, include the health effects language for Giardia lamblia, viruses, heterotrophic plate count bacteria, Legionella, and Cryptosporidium listed in Appendix A.

- Filter Backwash Recycle Rule (FBRR; if using conventional or direct filtration). If you violated the TT discussed in Checklist Item 4, include the health effects language for Giardia lamblia, viruses, heterotrophic plate count bacteria, Legionella, and Cryptosporidium listed in Appendix A.

- Long-Term 2 Enhanced Surface Water Treatment Rule (LT2ESWTR). If you violated the TT discussed in Checklist Item 4, see the example in Appendix F.

- Ground Water Rule (GWR). If you violated the TT discussed in Checklist Item 4, see the example in Appendix F.

- Stage 1 Disinfectants and Disinfection Byproducts Rule (Stage 1 D/DBPR; if using conventional filtration). If you violated the TT discussed in Checklist Item 4, include the health effects language for control of DBP precursors (TOC) listed in Appendix A.

- Lead and copper control requirements. If you violated the TT discussed in Checklist Item 4, must include the health effects language for lead or copper listed in Appendix A.

- Acrylamide and Epichlorohydrin. If you violated the TT discussed in Checklist Item 4, include the health effects language for acrylamide or epichlorohydrin listed in Appendix A.

- State Revised Total Coliform Rule (RTCR). If you violated this TT discussed in Checklist Item 4, include the health effects language listed in Appendix A, as appropriate.
Note: The State Water Board recommends that systems include TT violations listed here in a table adjacent to the main detected contaminant table. See Checklist Item 4 for more discussion on presenting TT violations.

- Monitoring and reporting of compliance data. If your system failed to take the sample on time (i.e., failure to monitor), the CCR should say “health effects unknown”. If your system took the samples accurately and on-time, but mailed the results late, you do not need to discuss health effects.

- Recordkeeping requirements.

- Special monitoring requirements under Cal. Code Regs., Title 22, § 64449(b)(2) and (g).

- Terms of a variance, an exemption, or an administrative or judicial order.

##### Ground Water Rule – Special Notice Requirements

The GWR requires CWSs and NTNCWSs using groundwater to provide special notice in their CCR for the following two situations:

###### Special Notice for Uncorrected Significant Deficiencies

If you are a groundwater system that receives notice from the State Water Board of a significant deficiency, you must inform your customers of any significant deficiencies that are not corrected by December 31st of the year covered by your CCR. The CCR must include the following information:

- The nature of the significant deficiency and the date it was identified by the State Water Board;

- The State Water Board-approved plan and schedule for correction, including interim measures, progress to date, and any interim measures completed.

You must continue to inform your customers annually until the State Water Board determines the significant deficiency is corrected.

Note: The State Water Board may also require you to include in your CCR significant deficiencies that were corrected by the end of the calendar year. If you are directed by the State Water Board to do this, you must inform your customers of the significant deficiency, how it was corrected, and the date it was corrected in that year’s CCR.

###### Special Notice for a Fecal Indicator-Positive Groundwater Source Sample

If you are a groundwater system that receives notice from a laboratory of a fecal indicator-positive groundwater source sample and the sample is not invalidated by the State Water Board, you must inform your customers in the next CCR. The CCR must include the following information for a fecal indicator-positive groundwater source sample:

- The source of the fecal contamination (if it is known) and the date(s) of the fecal indicator-positive source sample;

- If the fecal contamination has been addressed as prescribed by the requirements of the GWR [Cal. Code Regs., Title 22, § 64430, which incorporates by reference the federal GWR, i.e., 40 CFR section 141.403(a)] and the date the contamination was addressed;

- For fecal contamination that has not been addressed, the State Water Board-approved plan and schedule for correction, including interim measures, progress to date, and any interim measures completed;

- The health effects language for fecal indicators (E. coli, enterococci or coliphage), as provided in Appendix A.

Since fecal indicator-positive groundwater source samples must be included in the detected contaminant table, this special notice language can be included below the table or elsewhere in the report. Appendix F contains an example on how to present fecal indicator-positive groundwater source samples and the special notice text in a CCR.

You must continue to inform customers annually until the fecal contamination in the groundwater source is addressed as prescribed by the requirements of the GWR.

##### State Revised Total Coliform Rule – Language for Level 1 and Level 2 Assessment Requirements and Treatment Technique Violations, E. coli MCL Violation, and E. coli Detection

###### Level 1 and Level 2 Assessment Requirements and Treatment Technique Violations

Consistent with Cal. Code Regs., Title 22, § 64481(n)(1), water systems that had to comply with a Level 1 or Level 2 assessment requirement due to an E. coli violation and not due to an E. coli violation in 2022 are required to include information on the number of assessments required and completed, corrective actions required and completed, reasons for conducting assessment and corrective actions, and whether the water system failed to complete any required assessments or corrective actions.

###### Language to Use When a Level 1 or Level 2 Assessment Requirement Was Not Due to an E. coli MCL Violation

If your system was required to comply with a Level 1 or Level 2 Assessment requirement that was not due to an E. coli MCL violation, the CCR must include the following language in Cal. Code Regs., Title 22, Table 64481-A, filling in the blanks accordingly.

Coliforms are bacteria that occur naturally in the environment and are used as an indicator that other, potentially harmful, waterborne organisms may be present or that a potential pathway exists through which contamination may enter the drinking water distribution system. We found coliforms indicating the need to look for potential problems in water treatment or distribution. When this occurs, we are required to conduct assessment(s) to identify problems and to correct any problems that were found during these assessments. The water system shall include the following statements, as appropriate: Because we found coliforms during sampling, we were required to conduct [INSERT NUMBER OF LEVEL 1 ASSESSMENTS] assessment(s) of the system, also known as a Level 1 assessment, to identify possible sources of contamination. [INSERT NUMBER OF LEVEL 1 ASSESSMENTS] Level 1 assessment(s) were completed. In addition, we were required to take [INSERT NUMBER OF CORRECTIVE ACTIONS] corrective actions and we completed [INSERT NUMBER OF CORRECTIVE ACTIONS] of these actions. Because we found coliforms during sampling, we were required to conduct [INSERT NUMBER OF LEVEL 2 ASSESSMENTS] detailed assessments, also known as a Level 2 assessment to identify possible sources of contamination. [INSERT NUMBER OF LEVEL 2 ASSESSMENTS] Level 2 assessments were completed. In addition, we were required to take [INSERT NUMBER OF CORRECTIVE ACTIONS] corrective actions and we completed [INSERT NUMBER OF CORRECTIVE ACTIONS] of these actions.

If your system failed to complete all the required assessments or correct all identified sanitary defects, your system is in violation of the treatment technique requirement and shall include the following statements, as appropriate: [Cal. Code Regs., Title 22, § 64481(n)(1)].

During the past year we failed to conduct all the required assessment(s). During the past year we failed to correct all identified defects that were found during the assessment.

###### Language to Use When a Level 2 Assessment Requirement Was Due to an E. coli MCL Violation

If your system was required to comply with a Level 2 Assessment requirement that is due to an E. coli MCL violation, the CCR must include the following language in Cal. Code Regs., Title 22, Table 64481-B, filling in the blanks accordingly [Cal. Code Regs., Title 22, § 64481(n)(2)].

E. coli are bacteria whose presence indicates that the water may be contaminated with human or animal wastes. Human pathogens in these wastes can cause short-term effects, such as diarrhea, cramps, nausea, headaches, or other symptoms. They may pose a greater health risk for infants, young children, the elderly, and people with severely compromised immune systems. We found E. coli bacteria, indicating the need to look for potential problems in water treatment or distribution. When this occurs, we are required to conduct assessment(s), also known as a Level 2 assessment, to identify problems and to correct any problems that were found during these assessments. We were required to complete a detailed assessment of our water system, also known as a Level 2 assessment, because we found E. coli in our water system. In addition, we were required to take [INSERT NUMBER OF CORRECTIVE ACTIONS] corrective actions and we completed [INSERT NUMBER OF CORRECTIVE ACTIONS] of these actions.

If your system failed to complete all the required assessments or correct all identified sanitary defects, your system is in violation of the treatment technique requirement and shall include the following statements, as appropriate [Cal. Code Regs., Title 22, § 64481(n)(2)].

We failed to conduct the required assessment. We failed to correct all defects that were identified during the assessment.

###### E. coli MCL Violation

If your system detects E. coli and has violated the E. coli MCL, your system shall include one or more the following statements to describe any noncompliance, as applicable [Cal. Code Regs., Title 22, § 64481(n)(3)(A)-(D)]:

We had an E. coli-positive repeat sample following a total coliform-positive routine sample. We had a total coliform-positive repeat sample following an E. coli-positive routine sample. We failed to take all required repeat samples following an E. coli-positive routine sample. We failed to test for E. coli when any repeat sample tested positive for total coliform.

###### E. coli Detection

If your system detects E. coli and has not violated the E. coli MCL, include a statement that explains that although you have detected E. coli, your system is not in violation of the E. coli MCL [Cal. Code Regs., Title 22, § 64481(n)(4)].

#### Item 7: Variances and Exemptions

If your system operated under a variance or exemption at any time during the year covered by the CCR, include an explanation of the reasons for the variance or exemption, the date that it was issued, why it was granted, when it is up for renewal, and a status report on what the system is doing to remedy the problem (e.g., install treatment, find alternative sources of water, etc.) or otherwise comply with the terms and schedules of the variance or exemption. Also, tell your customers how they may participate in the review or renewal of the variance or exemption.

#### Item 8: Educational Information

##### For all CCRs

Every CCR must contain certain educational information, prominently displayed somewhere in the report.

Your CCR must contain basic information about drinking water contaminants. Use the following mandatory language.

Both tap water and bottled water come from rivers, lakes, streams, ponds, reservoirs, springs, and wells. As water travels over the surface of the land or through the ground, it dissolves naturally-occurring minerals and, in some cases, radioactive material. The water can also pick up and transport substances resulting from the presence of animals or from human activity. These substances are also called contaminants. Contaminants are any physical, chemical, biological, or radiological substance or matter in water. Contaminants that may be present in source water include: - Microbial contaminants, such as viruses and bacteria, that may come from sewage treatment plants, septic systems, agricultural livestock operations, and wildlife. - Inorganic contaminants, such as salts and metals, that can be occur naturally in the soil or groundwater or may result from urban stormwater runoff, industrial or domestic wastewater discharges, oil and gas production, mining, or farming. - Pesticides and herbicides, that may come from a variety of sources such as agriculture, urban stormwater runoff, and residential uses. - Organic chemical contaminants, including synthetic and volatile organic chemicals, that are byproducts of industrial processes and petroleum production, and can also come from gas stations, urban stormwater runoff, agricultural application, and septic systems. - Radioactive contaminants, that can occur naturally or be the result of oil and gas production and mining activities. To protect public health, the U.S. Environmental Protection Agency (U.S. EPA) and the State Water Resources Control Board (State Water Board) prescribe regulations that limit the amount of certain contaminants in tap water provided by public water systems. U.S. Food and Drug Administration regulations and California law also establish limits for contaminants in bottled water that provide the same protection for public health.

The following mandatory statement is a brief explanation regarding contaminants that may reasonably be expected to be found in drinking water, including bottled water.

Drinking water, including bottled water, may reasonably be expected to contain at least small amounts of some contaminants. The presence of contaminants does not necessarily mean that water poses a health risk. More information about contaminants and potential health effects can be obtained by contacting the U.S. EPA’s Safe Drinking Water Hotline (1-800-426-4791) or visiting the website epa.gov/safewater.

The next mandatory statement informs customers that some people may be more vulnerable to contaminants in drinking water than the general population and encourages those who may be particularly at risk from infection to seek advice from their health care provider.

Some people may be more vulnerable to contaminants in drinking water than the general population. Immuno-compromised persons such as persons with cancer undergoing chemotherapy, persons who have undergone organ transplants, people with HIV/AIDS or other immune system disorders, some elderly, and infants can be particularly at risk from infections. These people should seek advice about drinking water from their health care providers. U.S. EPA/Centers for Disease Control (CDC) guidelines on appropriate means to lessen the risk of infection by Cryptosporidium and other microbial contaminants are available from the Safe Drinking Water Hotline (1-800-426-4791) or the website epa.gov/safewater.

##### Other Educational Information

You are not limited to providing only the required information in your report. You may want to consider including:

- An explanation of (or include a diagram of) your system’s treatment processes;

Note: Although the CCR regulations do not require you to provide treatment information in your CCR, the State Water Board strongly recommends that you use the CCR as an opportunity to inform your consumers about the treatment processes applied to their water, particularly fluoridation if it is used. Even though there is a fluoridation regulation that requires you to inform your consumers when fluoridation is initiated, or taken off line for an extended period, it is quite likely that many consumers are unaware that fluoride is being added or being received from another water system. This information could potentially affect decisions your consumers make regarding fluoride supplements and treatments. You may want to provide the address for the State Water Board’s fluoridation website, where consumers may obtain more information about fluoridation, oral health, and current issues (http://www.waterboards.ca.gov/drinking_water/certlic/drinkingwater/Fluoridation.shtml).

- Source water protection tips (refer to Appendix I for example language regarding source water protection tips);

- Water and energy conservation tips (refer to Appendix I for example language regarding water conservation tips);

- The cost of making the water safe to drink including the cost of sustaining the infrastructure;

- Efforts your system has made to promote “green infrastructure” (e.g., stormwater pollution prevention measures);

- A statement from the mayor or general manager;

- Information to educate customers about taste and odor issues, affiliations with programs such as the Partnership for Safe Water, opportunities for public participation, etc.

You may want to provide the address for U.S. EPA’s drinking water website (https://www.epa.gov/ground-water-and-drinking-water). The only limitation on this information is that it must not interfere with the educational purpose of the CCR.

#### Item 9: Six-Month Update

If your system serves more than 10,000 people, you must also distribute the report again by December 31, 2027. The second report must include a six-month update if:

- A violation or action level exceedance occurred between January 1 and June 30 of the current year (2027);

- Results from required UCMR monitoring were received.

If you are required to include a six-month update, it must contain:

- A short description of the nature of the 6-month update and the twice yearly delivery;

- If a MCL, MRDL, or treatment technique violation was received:

  - The required contaminant section information (see Checklist Item 4)

  - A readily understandable explanation of the violation including:

    - The length of the violation

    - The potential adverse health effects

    - Actions taken by the system to address the violation

    - Timeframe the system expects to complete those actions

    - To describe the potential health effects, the system must use the relevant language in Appendix A

- If any other violation was received:

  - A summary of the related information required in the CCR

- If a system exceeded the lead action level following monitoring conducted between January 1 and June 30 of the current year:

  - The 90th percentile value of the most recent round of sampling,

  - The number of sites sampled,

  - The number of sites exceeding the action level,

  - If any corrective action has been required during the monitoring period covered by the report, clearly identify the exceedance,

  - a clear and readily understandable explanation of the exceedance,

  - the steps consumers can take to reduce their exposure to lead in drinking water, and

  - a description of any corrective actions the system has or will take to address the exceedance.

- If systems monitoring under the UCMR become aware of results for samples (except Cryptosporidium) collected during the reporting year but were not included in the reports distributed by July 1:

  - The average and range of results for detected contaminants.

## What should the CCR look like?

You do not need a fancy computer or a graphic designer to produce a CCR that is easy to read and inviting to your customers. The best way to design your CCR is to spend some time looking at other CCRs. See what catches your eye and copy it. A few things to consider:

- Limit wordiness – write short sentences and keep your paragraphs short as well.

- Do not make your text size too small. You might want to squeeze a few extra sentences in your CCR, but if you add too much, people might ignore the entire report.

- Give a draft of your CCR to relatives or friends who are not drinking water experts and ask them if it makes sense. Ask customers for their comments when you publish the CCR.

- Do not distract from your main message with graphics and/or pictures that do not complement your message.

- Be as simple, truthful, and straight forward as possible. Avoid acronyms, initials, and jargon.

- Consider printing the CCR on recycled paper and taking other steps to make the CCR “environmentally friendly”. If you hope to get your customers involved in protecting source water, set a good example for them.

## How must the CCR be distributed?

### General

You must directly deliver a copy of your CCR to each of your customers and make a good faith effort to get CCRs to non-bill-paying consumers. Use at least one of the following forms of delivery:

- Mail or hand deliver a copy of the report

- Mail a notification that the report is available on a website via a direct link

- Email a direct link or electronic version of the report

You may include your CCR with water bills, if feasible, or you may send it as a separate mailer. Sending your CCR as a separate mailer will likely be more effective, and you will reach renters who may not receive water bills directly.

If using an electronic delivery method, you must provide a paper copy of the report to any consumer upon request, and the notification method must prominently display directions for requesting such a copy. You must make your CCR available to the public upon request, and you should make a reasonable effort to provide the reports in an accessible format to anyone who requests an accommodation.

If you choose to electronically deliver the reports by posting the report to a website and providing a notification either by mail or email:

- The report must be publicly available on the website at the time the notification is made;

- Notifications must prominently display the link and include an explanation of the nature of the link;

- You may use a webpage to convey the information required in the CCR.

Systems that serve 50,000 or more people must post their CCRs on a publicly-accessible site on the internet. We encourage other systems to post their CCRs as well. Many local governments have sites where you can post your CCR, even if your system itself does not have a website. U.S. EPA provides a mechanism that allows systems to link their CCR to the U.S. EPA website (https://www.epa.gov/ccr/how-water-systems-comply-ccr-requirements).

If you use a publicly available website to provide reports, you must maintain public access to the report for at least 3 years. All systems must internally retain copies of their CCRs for at least 3 years.

### Consumers not receiving water bills

It is in your system’s interest to spread the word about the quality of its water. Since many consumers of your water may not receive bills (people such as apartment renters or workers), you must make “good faith” efforts to reach non-bill paying consumers. A “good faith” effort means selecting a mix of methods to reach the broadest possible range of persons served your system’s water. Those options include but are not limited to:

- Posting the CCR on the internet;

- Mailing the CCR or postcards with links to the CCR to all service addresses and/or postal customers;

- Using an opt in notification system to send emails and/or texts with links to the CCR to interested consumers;

- Advertising the availability of the CCR in news media (e.g., newspapers, TV, and radio) and/or on social media;

- Publishing the complete CCR in a local newspaper or newsletter;

- Posting the CCR or notice of availability with links (or equivalent, such QE codes) in public places such as cafeterias, lunchrooms, and lobbies of public buildings, libraries, churches, and schools;

- Delivering multiple copies of the CCRs for distribution by single-billed customers such as apartment buildings or large private employers;

- Delivering the CCR to community organizations;

- Holding a public meeting to educate consumers on the CCR.

Where a system is aware that it serves a substantial number of non-bill paying consumers, the system is encouraged to directly deliver the reports or notices of availability of the reports to service addresses.

Where a system is aware of a substantial number of bill-paying consumers without access to electronic forms of the report, the system should use at least one non-electronic form of delivery.

### State and local agencies and media outlets

No later than 10 days after the CCR distribution deadline(s) that apply to your system (July 1 and/or December 31), you must send the following to the State Water Board or LPA, depending on who oversees your regulatory compliance:

- A copy of the CCR;

- A certification (see Appendix G for a sample form) that you distributed the CCR and that its information is correct and consistent with the compliance monitoring data previously submitted to the State Water Board;

- A copy of the system’s plan for providing assistance to consumers with limited English proficiency, if the current version of the plan has not yet been submitted.

In addition, if your system is investor-owned, send a copy of the CCR to the California Public Utilities Commission by the CCR distribution deadline(s) that apply to your system (July 1 and/or December 31). We also encourage you to send copies to local TV and radio stations and newspapers.

### Electronic delivery

The State Water Board allows electronic delivery of the CCR. Suggested delivery methods, examples, and the certification form to use are available on the State Water Board’s website (http://www.swrcb.ca.gov/drinking_water/certlic/drinkingwater/CCR.shtml).

## How long must the CCR be kept?

Keep your report on file for no less than three years.

## APPENDIX A: Regulated Contaminants with Primary Drinking Water Standards

### Microbiological Contaminants

| Contaminant | Traditional MCL | To convert for CCR, multiply by | MCL in CCR units | PHG (MCLG) in CCR units | Major sources in Drinking Water | Health Effects Language |
| --- | --- | --- | --- | --- | --- | --- |
| E. coli (State Revised Total Coliform Rule) | MCL Violations: Routine and repeat samples are total coliform-positive and either is E. coli positive or system fails to take repeat samples following E. coli-positive routine sample or system fails to analyze total coliform-positive repeat sample for E. coli. |  |  | (0) | Human and animal fecal waste | E. coli are bacteria whose presence indicates that the water may be contaminated with human or animal wastes. Human pathogens in these wastes can cause short-term effects, such as diarrhea, cramps, nausea, headaches, or other symptoms. They may pose a greater health risk for infants, young children, some of the elderly, and people with severely-compromised immune systems. |
| Total Coliform Bacteria | TT | N/A | TT | N/A | Naturally present in the environment | Coliforms are bacteria that occur naturally in the environment and are used as an indicator that other, potentially harmful, waterborne organisms may be present or that a potential pathway exists through which contamination may enter the drinking water distribution system. We found coliforms indicating the need to look for potential problems in water treatment or distribution. When this occurs, we are required to conduct assessment(s) to identify problems and to correct any problems that were found. |
| Fecal Indicator E. coli (Ground Water Rule) | 0 | N/A | 0 | (0) | Human and animal fecal waste | E. coli are bacteria whose presence indicates that the water may be contaminated with human or animal wastes. Human pathogens in these wastes can cause short-term effects, such as diarrhea, cramps, nausea, headaches, or other symptoms. They may pose a greater health risk for infants, young children, the elderly, and people with severely-compromised immune systems. |
| Fecal Indicators (enterococci or coliphage) (Ground Water Rule) | TT | N/A | TT | N/A | Human and animal fecal waste | Fecal indicators are microbes whose presence indicates that the water may be contaminated with human or animal wastes. Microbes in these wastes can cause short-term effects, such as diarrhea, cramps, nausea, headaches, or other symptoms. They may pose a special health risk for infants, young children, some of the elderly, and people with severely compromised immune systems. |
| Seasonal System Treatment Technique Violations | TT | N/A | TT | N/A | N/A | When this violation includes failure to monitor for total coliforms or E. coli prior to serving water to the public, the mandatory language found at 22 California Code of Regulations section 64465(a)(11) shall be used. When the violation includes failure to complete other actions, the appropriate elements found in sections 64465(a)(1) through (10) to describe the violation shall be used. |
| Turbidity | TT | N/A | TT | N/A | Soil runoff | Turbidity has no health effects. However, high levels of turbidity can interfere with disinfection and provide a medium for microbial growth. Turbidity may indicate the presence of disease-causing organisms. These organisms include bacteria, viruses, and parasites that can cause symptoms such as nausea, cramps, diarrhea, and associated headaches. |
| Giardia lamblia, Viruses, Heterotrophic Plate Count Bacteria, Legionella, Cryptosporidium | TT | TT | TT | HPC = N/A; Others = (0) | Naturally present in the environment | Inadequately treated water may contain disease-causing organisms. These organisms include bacteria, viruses, and parasites that can cause symptoms such as nausea, cramps, diarrhea, and associated headaches. |

TT = Treatment Technique

### Radioactive Contaminants

| Contaminant (CCR units) | Traditional MCL | To convert for CCR, multiply by | MCL in CCR units | PHG (MCLG) in CCR units | Major Sources in Drinking Water | Health Effects Language |
| --- | --- | --- | --- | --- | --- | --- |
| Gross Beta Particle Activity (pCi/L) | 50 | N/A | 50 | (0) | Decay of natural and man-made deposits | Certain minerals are radioactive and may emit forms of radiation known as photons and beta radiation. Some people who drink water containing beta and photon emitters in excess of the MCL over many years may have an increased risk of getting cancer. |
| Strontium-90 (pCi/L) | 8 | N/A | 8 | 0.35 | Decay of natural and man-made deposits | Some people who drink water containing strontium-90 in excess of the MCL over many years may have an increased risk of getting cancer. |
| Tritium (pCi/L) | 20,000 | N/A | 20,000 | 400 | Decay of natural and man-made deposits | Some people who drink water containing tritium in excess of the MCL over many years may have an increased risk of getting cancer. |
| Gross Alpha Particle Activity (pCi/L) | 15 | N/A | 15 | (0) | Erosion of natural deposits | Certain minerals are radioactive and may emit a form of radiation known as alpha radiation. Some people who drink water containing alpha emitters in excess of the MCL over many years may have an increased risk of getting cancer. |
| Combined Radium (pCi/L) | 5 | N/A | 5 | (0) | Erosion of natural deposits | Some people who drink water containing radium 226 or 228 in excess of the MCL over many years may have an increased risk of getting cancer. |
| Total Radium (pCi/L) (for NTNC water systems) | 5 | N/A | 5 | N/A | Erosion of natural deposits | Some people who drink water containing radium 223, 224, or 226 in excess of the MCL over many years may have an increased risk of getting cancer. |
| Uranium (pCi/L) | 20 | N/A | 20 | 0.43 | Erosion of natural deposits | Some people who drink water containing uranium in excess of the MCL over many years may have kidney problems or an increased risk of getting cancer. |

### Inorganic Contaminants

| Contaminant (CCR units) | Traditional MCL in mg/L | To convert for CCR, multiply by | MCL in CCR units | PHG (MCLG) in CCR units | Major Sources in Drinking Water | Health Effects Language |
| --- | --- | --- | --- | --- | --- | --- |
| Aluminum (mg/L) | 1 | - | 1 | 0.6 | Erosion of natural deposits; residue from some surface water treatment processes | Some people who drink water containing aluminum in excess of the MCL over many years may experience short-term gastrointestinal tract effects. |
| Antimony (µg/L) | 0.006 | 1,000 | 6 | 1 | Discharge from petroleum refineries; fire retardants; ceramics; electronics; solder | Some people who drink water containing antimony in excess of the MCL over many years may experience increases in blood cholesterol and decreases in blood sugar. |
| Arsenic (µg/L) | 0.010 | 1,000 | 10 | 0.004 | Erosion of natural deposits; runoff from orchards; glass and electronics production wastes | Some people who drink water containing arsenic in excess of the MCL over many years may experience skin damage or circulatory system problems, and may have an increased risk of getting cancer. |
| Asbestos (MFL) | 7 MFL | - | 7 | 7 | Internal corrosion of asbestos cement water mains; erosion of natural deposits | Some people who drink water containing asbestos in excess of the MCL over many years may have an increased risk of developing benign intestinal polyps. |
| Barium (mg/L) | 1 | - | 1 | 2 | Discharges of oil drilling wastes and from metal refineries; erosion of natural deposits | Some people who drink water containing barium in excess of the MCL over many years may experience an increase in blood pressure. |
| Beryllium (µg/L) | 0.004 | 1,000 | 4 | 1 | Discharge from metal refineries, coal-burning factories, and electrical, aerospace, and defense industries | Some people who drink water containing beryllium in excess of the MCL over many years may develop intestinal lesions. |
| Cadmium (µg/L) | 0.005 | 1,000 | 5 | 0.04 | Internal corrosion of galvanized pipes; erosion of natural deposits; discharge from electroplating and industrial chemical factories, and metal refineries; runoff from waste batteries and paints | Some people who drink water containing cadmium in excess of the MCL over many years may experience kidney damage. |
| Chromium, hexavalent (µg/L) | 0.010 See Table 64432-B within Title 22 of the California Code of Regulations for applicable compliance date. | 1,000 | 10 | 0.02 | Erosion of natural deposits; transformation of naturally occurring trivalent chromium to hexavalent chromium by natural processes and human activities such as discharges from electroplating factories, leather tanneries, wood preservation, chemical synthesis, refractory production, and textile manufacturing facilities. | Some people who drink water containing hexavalent chromium in excess of the MCL over many years may have an increased risk of getting cancer. |
| Chromium, total (µg/L) | 0.05 | 1,000 | 50 | (100) | Discharge from steel and pulp mills and chrome plating; erosion of natural deposits | Some people who use water containing chromium in excess of the MCL over many years may experience allergic dermatitis. |
| Copper (mg/L) | AL = 1.3 | - | AL = 1.3 | 0.3 | Internal corrosion of household plumbing systems; erosion of natural deposits; leaching from wood preservatives | Copper is an essential nutrient, but some people who drink water containing copper in excess of the action level over a relatively short amount of time may experience gastrointestinal distress. Some people who drink water containing copper in excess of the action level over many years may suffer liver or kidney damage. People with Wilson’s Disease should consult their personal doctor. |
| Cyanide (µg/L) | 0.15 | 1,000 | 150 | 150 | Discharge from steel/metal, plastic and fertilizer factories | Some people who drink water containing cyanide in excess of the MCL over many years may experience nerve damage or thyroid problems. |
| Fluoride (mg/L) | 2.0 | - | 2.0 | 1 | Erosion of natural deposits; water additive that promotes strong teeth; discharge from fertilizer and aluminum factories | Some people who drink water containing fluoride in excess of the federal MCL of 4 mg/L over many years may get bone disease, including pain and tenderness of the bones. Children who drink water containing fluoride in excess of the state MCL of 2 mg/L may get mottled teeth. Mottling, also known as dental fluorosis, may include brown staining and/or pitting of the teeth, and occurs only in developing teeth before they erupt from the gums. |
| Lead (µg/L) | AL = 0.015 | 1,000 | AL = 15 | 0.2 | Corrosion of household water plumbing systems and service lines connecting buildings to water mains; discharges from industrial manufacturers; erosion of natural deposits | There is no safe level of lead in drinking water. Exposure to lead in drinking water can cause serious health effects in all age groups, especially pregnant people, infants (both formula-fed and breastfed), and young children. Some of the health effects to infants and children include decreases in IQ and attention span. Lead exposure can also result in new or worsened learning and behavior problems. The children of persons who are exposed to lead before or during pregnancy may be at increased risk of these harmful health effects. Adults have increased risks of heart disease, high blood pressure, kidney or nervous system problems. Contact your health care provider for more information about your risks. |
| Mercury [Inorganic] (µg/L) | 0.002 | 1,000 | 2 | 1.2 | Erosion of natural deposits; discharge from refineries and factories; runoff from landfills and cropland | Some people who drink water containing mercury in excess of the MCL over many years may experience kidney damage, mental disturbances, or impaired physical coordination, speech and hearing. |
| Nickel (µg/L) | 0.1 | 1,000 | 100 | 12 | Erosion of natural deposits; discharge from metal factories | Some people who drink water containing nickel in excess of the MCL over many years may experience liver and heart effects. |
| Nitrate (mg/L) | 10 (as N) | - | 10 (as N) | 10 (as N) | Runoff and leaching from fertilizer use; leaching from septic tanks and sewage; erosion of natural deposits | Infants below the age of six months who drink water containing nitrate in excess of the MCL may quickly become seriously ill and, if untreated, may die because high nitrate levels can interfere with the capacity of the infant’s blood to carry oxygen. Symptoms include shortness of breath and blueness of the skin. High nitrate levels may also affect the oxygen-carrying ability of the blood of pregnant women. |
| Nitrite (mg/L) | 1 (as N) | - | 1 (as N) | 1 (as N) | Runoff and leaching from fertilizer use; leaching from septic tanks and sewage; erosion of natural deposits | Infants below the age of six months who drink water containing nitrite in excess of the MCL may quickly become seriously ill and, if untreated, may die. Symptoms include shortness of breath and blueness of the skin. |
| Perchlorate (µg/L) | 0.006 | 1,000 | 6 | 1 | Perchlorate is an inorganic chemical used in solid rocket propellant, fireworks, explosives, flares, matches, and a variety of industries. It usually gets into drinking water as a result of environmental contamination from historic aerospace or other industrial operations that used or use, store, or dispose of perchlorate and its salts. | Perchlorate has been shown to interfere with uptake of iodide by the thyroid gland, and to thereby reduce the production of thyroid hormones, leading to adverse effects associated with inadequate hormone levels. Thyroid hormones are needed for normal prenatal growth and development of the fetus, as well as for normal growth and development in the infant and child. In adults, thyroid hormones are needed for normal metabolism and mental function. |
| Selenium (µg/L) | 0.05 | 1,000 | 50 | 30 | Discharge from petroleum, glass, and metal refineries; erosion of natural deposits; discharge from mines and chemical manufacturers; runoff from livestock lots (feed additive) | Selenium is an essential nutrient. However, some people who drink water containing selenium in excess of the MCL over many years may experience hair or fingernail losses, numbness in fingers or toes, or circulation system problems. |
| Thallium (µg/L) | 0.002 | 1,000 | 2 | 0.1 | Leaching from ore-processing sites; discharge from electronics, glass, and drug factories | Some people who drink water containing thallium in excess of the MCL over many years may experience hair loss, changes in their blood, or kidney, intestinal, or liver problems. |

### Synthetic Organic Contaminants including Pesticides and Herbicides

| Contaminant (CCR units) | Traditional MCL in mg/L | To convert for CCR, multiply by | MCL in CCR units | PHG (MCLG) in CCR units | Major Sources in Drinking Water | Health Effects Language |
| --- | --- | --- | --- | --- | --- | --- |
| 2,4-D (µg/L) | 0.07 | 1,000 | 70 | 20 | Runoff from herbicide used on row crops, range land, lawns, and aquatic weeds | Some people who use water containing the weed killer 2,4-D in excess of the MCL over many years may experience kidney, liver, or adrenal gland problems. |
| 2,4,5-TP [Silvex] (µg/L) | 0.05 | 1,000 | 50 | 3 | Residue of banned herbicide | Some people who drink water containing Silvex in excess of the MCL over many years may experience liver problems. |
| Acrylamide | TT | - | TT | (0) | Added to water during sewage/wastewater treatment | Some people who drink water containing high levels of acrylamide over a long period of time may experience nervous system or blood problems, and may have an increased risk of getting cancer. |
| Alachlor (µg/L) | 0.002 | 1,000 | 2 | 4 | Runoff from herbicide used on row crops | Some people who use water containing alachlor in excess of the MCL over many years may experience eye, liver, kidney, or spleen problems, or experience anemia, and may have an increased risk of getting cancer. |
| Atrazine (µg/L) | 0.001 | 1,000 | 1 | 0.15 | Runoff from herbicide used on row crops and along railroad and highway right-of-ways | Some people who use water containing atrazine in excess of the MCL over many years may experience cardiovascular system problems or reproductive difficulties. |
| Bentazon (µg/L) | 0.018 | 1,000 | 18 | 200 | Runoff/leaching from herbicide used on beans, peppers, corn, peanuts, rice, and ornamental grasses | Some people who drink water containing bentazon in excess of the MCL over many years may experience prostate and gastrointestinal effects. |
| Benzo(a)pyrene [PAH] (ng/L) | 0.0002 | 1,000,000 | 200 | 7 | Leaching from linings of water storage tanks and distribution mains | Some people who use water containing benzo(a)pyrene in excess of the MCL over many years may experience reproductive difficulties and may have an increased risk of getting cancer. |
| Carbofuran (µg/L) | 0.018 | 1,000 | 18 | 0.7 | Leaching of soil fumigant used on rice and alfalfa, and grape vineyards | Some people who use water containing carbofuran in excess of the MCL over many years may experience problems with their blood, or nervous or reproductive system problems. |
| Chlordane (ng/L) | 0.0001 | 1,000,000 | 100 | 30 | Residue of banned insecticide | Some people who use water containing chlordane in excess of the MCL over many years may experience liver or nervous system problems, and may have an increased risk of getting cancer. |
| Dalapon (µg/L) | 0.2 | 1,000 | 200 | 790 | Runoff from herbicide used on rights-of-way, and crops and landscape maintenance | Some people who drink water containing dalapon in excess of the MCL over many years may experience minor kidney changes. |
| Di(2-ethylhexyl) Adipate (µg/L) | 0.4 | 1,000 | 400 | 200 | Discharge from chemical factories | Some people who drink water containing di(2-ethylhexyl) adipate in excess of the MCL over many years may experience weight loss, liver enlargement, or possible reproductive difficulties. |
| Di(2-ethylhexyl) Phthalate (µg/L) | 0.004 | 1,000 | 4 | 12 | Discharge from rubber and chemical factories; inert ingredient in pesticides | Some people who use water containing di(2-ethylhexyl) phthalate well in excess of the MCL over many years may experience liver problems or reproductive difficulties, and may have an increased risk of getting cancer. |
| Dibromochloropropane [DBCP] (ng/L) | 0.0002 | 1,000,000 | 200 | 3 | Banned nematocide that may still be present in soils due to runoff/leaching from former use on soybeans, cotton, vineyards, tomatoes, and tree fruit | Some people who use water containing DBCP in excess of the MCL over many years may experience reproductive difficulties and may have an increased risk of getting cancer. |
| Dinoseb (µg/L) | 0.007 | 1,000 | 7 | 14 | Runoff from herbicide used on soybeans, vegetables, and fruits | Some people who drink water containing dinoseb in excess of the MCL over many years may experience reproductive difficulties. |
| Dioxin [2,3,7,8-TCDD] (pg/L) | 0.00000003 | 1,000,000,000 | 30 | 0.05 | Emissions from waste incineration and other combustion; discharge from chemical factories | Some people who use water containing dioxin in excess of the MCL over many years may experience reproductive difficulties and may have an increased risk of getting cancer. |
| Diquat (µg/L) | 0.02 | 1,000 | 20 | 6 | Runoff from herbicide use for terrestrial and aquatic weeds | Some people who drink water containing diquat in excess of the MCL over many years may get cataracts. |
| Endothall (µg/L) | 0.1 | 1,000 | 100 | 94 | Runoff from herbicide use for terrestrial and aquatic weeds; defoliant | Some people who drink water containing endothall in excess of the MCL over many years may experience stomach or intestinal problems. |
| Endrin (µg/L) | 0.002 | 1,000 | 2 | 0.3 | Residue of banned insecticide and rodenticide | Some people who drink water containing endrin in excess of the MCL over many years may experience liver problems. |
| Epichlorohydrin | TT | - | TT | (0) | Discharge from industrial chemical factories; impurity of some water treatment chemicals | Some people who drink water containing high levels of epichlorohydrin over a long period of time may experience stomach problems, and may have an increased risk of getting cancer. |
| Ethylene Dibromide [EDB] (ng/L) | 0.00005 | 1,000,000 | 50 | 10 | Discharge from petroleum refineries; underground gas tank leaks; banned nematocide that may still be present in soils due to runoff and leaching from grain and fruit crops | Some people who use water containing ethylene dibromide in excess of the MCL over many years may experience liver, stomach, reproductive system, or kidney problems, and may have an increased risk of getting cancer. |
| Glyphosate (µg/L) | 0.7 | 1,000 | 700 | 900 | Runoff from herbicide use | Some people who drink water containing glyphosate in excess of the MCL over many years may experience kidneys problems or reproductive difficulties. |
| Heptachlor (ng/L) | 0.00001 | 1,000,000 | 10 | 8 | Residue of banned insecticide | Some people who use water containing heptachlor in excess of the MCL over many years may experience liver damage and may have an increased risk of getting cancer. |
| Heptachlor Epoxide (ng/L) | 0.00001 | 1,000,000 | 10 | 6 | Breakdown of heptachlor | Some people who use water containing heptachlor epoxide in excess of the MCL over many years may experience liver damage, and may have an increased risk of getting cancer. |
| Hexachlorobenzene (µg/L) | 0.001 | 1,000 | 1 | 0.03 | Discharge from metal refineries and agricultural chemical factories; byproduct of chlorination reactions in wastewater | Some people who drink water containing hexachlorobenzene in excess of the MCL over many years may experience liver or kidney problems, or adverse reproductive effects, and may have an increased risk of getting cancer. |
| Hexachlorocyclo-pentadiene (µg/L) | 0.05 | 1,000 | 50 | 2 | Discharge from chemical factories | Some people who use water containing hexachlorocyclopentadiene in excess of the MCL over many years may experience kidney or stomach problems. |
| Lindane (ng/L) | 0.0002 | 1,000,000 | 200 | 32 | Runoff/leaching from insecticide used on cattle, lumber, and gardens | Some people who drink water containing lindane in excess of the MCL over many years may experience kidney or liver problems. |
| Methoxychlor (µg/L) | 0.03 | 1,000 | 30 | 0.09 | Runoff/leaching from insecticide used on fruits, vegetables, alfalfa, and livestock | Some people who drink water containing methoxychlor in excess of the MCL over many years may experience reproductive difficulties. |
| Molinate [Ordram] (µg/L) | 0.02 | 1,000 | 20 | 1 | Runoff/leaching from herbicide used on rice | Some people who use water containing molinate in excess of the MCL over many years may experience reproductive effects. |
| Oxamyl [Vydate] (µg/L) | 0.05 | 1,000 | 50 | 26 | Runoff/leaching from insecticide used on field crops, fruits and ornamentals, especially apples, potatoes, and tomatoes | Some people who drink water containing oxamyl in excess of the MCL over many years may experience slight nervous system effects. |
| PCBs [Polychlorinated Biphenyls] (ng/L) | 0.0005 | 1,000,000 | 500 | 90 | Runoff from landfills; discharge of waste chemicals | Some people who drink water containing PCBs in excess of the MCL over many years may experience changes in their skin, thymus gland problems, immune deficiencies, or reproductive or nervous system difficulties, and may have an increased risk of getting cancer. |
| Pentachlorophenol (µg/L) | 0.001 | 1,000 | 1 | 0.3 | Discharge from wood preserving factories, cotton and other insecticidal/herbicidal uses | Some people who use water containing pentachlorophenol in excess of the MCL over many years may experience liver or kidney problems, and may have an increased risk of getting cancer. |
| Picloram (µg/L) | 0.5 | 1,000 | 500 | 166 | Herbicide runoff | Some people who drink water containing picloram in excess of the MCL over many years may experience liver problems. |
| Simazine (µg/L) | 0.004 | 1,000 | 4 | 4 | Herbicide runoff | Some people who use water containing simazine in excess of the MCL over many years may experience blood problems. |
| Thiobencarb (µg/L) | 0.07 | 1,000 | 70 | 42 | Runoff/leaching from herbicide used on rice | Some people who use water containing thiobencarb in excess of the MCL over many years may experience body weight and blood effects. |
| Toxaphene (µg/L) | 0.003 | 1,000 | 3 | 0.03 | Runoff/leaching from insecticide used on cotton and cattle | Some people who use water containing toxaphene in excess of the MCL over many years may experience kidney, liver, or thyroid problems, and may have an increased risk of getting cancer. |
| 1,2,3-Trichloropropane [TCP] (ng/L) | 0.000005 | 1,000,000 | 5 | 0.7 | Discharge from industrial and agricultural chemical factories; leaching from hazardous waste sites; used as cleaning and maintenance solvent, paint and varnish remover, and cleaning and degreasing agent; byproduct during the production of other compounds and pesticides. | Some people who drink water containing 1,2,3-trichloropropane in excess of the MCL over many years may have an increased risk of getting cancer. |

### Volatile Organic Contaminants

| Contaminant (CCR units) | Traditional MCL in mg/L | To convert for CCR, multiply by | MCL in CCR units | PHG (MCLG) in CCR units | Major Sources in Drinking Water | Health Effects Language |
| --- | --- | --- | --- | --- | --- | --- |
| Benzene (µg/L) | 0.001 | 1,000 | 1 | 0.15 | Discharge from plastics, dyes and nylon factories; leaching from gas storage tanks and landfills | Some people who use water containing benzene in excess of the MCL over many years may experience anemia or a decrease in blood platelets, and may have an increased risk of getting cancer. |
| Carbon Tetrachloride (ng/L) | 0.0005 | 1,000,000 | 500 | 100 | Discharge from chemical plants and other industrial activities | Some people who use water containing carbon tetrachloride in excess of the MCL over many years may experience liver problems and may have an increased risk of getting cancer. |
| 1,2-Dichlorobenzene (µg/L) | 0.6 | 1,000 | 600 | 600 | Discharge from industrial chemical factories | Some people who drink water containing 1,2-dichlorobenzene in excess of the MCL over many years may experience liver, kidney, or circulatory system problems. |
| 1,4-Dichlorobenzene (µg/L) | 0.005 | 1,000 | 5 | 6 | Discharge from industrial chemical factories | Some people who use water containing 1,4-dichlorobenzene in excess of the MCL over many years may experience anemia, liver, kidney, or spleen damage, or changes in their blood. |
| 1,1-Dichloroethane (µg/L) | 0.005 | 1,000 | 5 | 3 | Extraction and degreasing solvent; used in manufacture of pharmaceuticals, stone, clay and glass products; fumigant | Some people who use water containing 1,1-dichloroethane in excess of the MCL over many years may experience nervous system or respiratory problems. |
| 1,2-Dichloroethane (ng/L) | 0.0005 | 1,000,000 | 500 | 400 | Discharge from industrial chemical factories | Some people who use water containing 1,2-dichloroethane in excess of the MCL over many years may have an increased risk of getting cancer. |
| 1,1-Dichloroethylene (µg/L) | 0.006 | 1,000 | 6 | 10 | Discharge from industrial chemical factories | Some people who use water containing 1,1-dichloroethylene in excess of the MCL over many years may experience liver problems. |
| cis-1,2-Dichloroethylene (µg/L) | 0.006 | 1,000 | 6 | 13 | Discharge from industrial chemical factories; major biodegradation byproduct of TCE and PCE groundwater contamination | Some people who use water containing cis-1,2-dichloroethylene in excess of the MCL over many years may experience liver problems. |
| trans-1,2-Dichloroethylene (µg/L) | 0.01 | 1,000 | 10 | 50 | Discharge from industrial chemical factories; minor biodegradation byproduct of TCE and PCE groundwater contamination | Some people who drink water containing trans-1,2-dichloroethylene in excess of the MCL over many years may experience liver problems. |
| Dichloromethane (µg/L) | 0.005 | 1,000 | 5 | 4 | Discharge from pharmaceutical and chemical factories; insecticide | Some people who drink water containing dichloromethane in excess of the MCL over many years may experience liver problems and may have an increased risk of getting cancer. |
| 1,2-Dichloropropane (µg/L) | 0.005 | 1,000 | 5 | 0.5 | Discharge from industrial chemical factories; primary component of some fumigants | Some people who use water containing 1,2-dichloropropane in excess of the MCL over many years may have an increased risk of getting cancer. |
| 1,3-Dichloropropene (ng/L) | 0.0005 | 1,000,000 | 500 | 200 | Runoff/leaching from nematocide used on croplands | Some people who use water containing 1,3-dichloropropene in excess of the MCL over many years may have an increased risk of getting cancer. |
| Ethylbenzene (µg/L) | 0.3 | 1,000 | 300 | 300 | Discharge from petroleum refineries; industrial chemical factories | Some people who use water containing ethylbenzene in excess of the MCL over many years may experience liver or kidney problems. |
| Methyl-tert-butyl ether (µg/L) | 0.013 | 1,000 | 13 | 13 | Leaking underground storage tanks; discharge from petroleum and chemical factories | Some people who use water containing methyl-tert-butyl ether in excess of the MCL over many years may have an increased risk of getting cancer. |
| Monochlorobenzene (µg/L) | 0.07 | 1,000 | 70 | 70 | Discharge from industrial and agricultural chemical factories and dry cleaning facilities | Some people who use water containing monochlorobenzene in excess of the MCL over many years may experience liver or kidney problems. |
| Styrene (µg/L) | 0.1 | 1,000 | 100 | 0.5 | Discharge from rubber and plastic factories; leaching from landfills | Some people who drink water containing styrene in excess of the MCL over many years may experience liver, kidney, or circulatory system problems. |
| 1,1,2,2-Tetrachloroethane (µg/L) | 0.001 | 1,000 | 1 | 0.1 | Discharge from industrial and agricultural chemical factories; solvent used in production of TCE, pesticides, varnish and lacquers | Some people who drink water containing 1,1,2,2-tetrachloroethane in excess of the MCL over many years may experience liver or nervous system problems. |
| Tetrachloroethylene (PCE) (µg/L) | 0.005 | 1,000 | 5 | 0.06 | Discharge from factories, dry cleaners, and auto shops (metal degreaser) | Some people who use water containing tetrachloroethylene in excess of the MCL over many years may experience liver problems, and may have an increased risk of getting cancer. |
| 1,2,4-Trichlorobenzene (µg/L) | 0.005 | 1,000 | 5 | 5 | Discharge from textile-finishing factories | Some people who use water containing 1,2,4-trichlorobenzene in excess of the MCL over many years may experience adrenal gland changes. |
| 1,1,1-Trichloroethane (µg/L) | 0.200 | 1,000 | 200 | 1000 | Discharge from metal degreasing sites and other factories; manufacture of food wrappings | Some people who use water containing 1,1,1-trichloroethane in excess of the MCL over many years may experience liver, nervous system, or circulatory system problems. |
| 1,1,2-Trichloroethane (µg/L) | 0.005 | 1,000 | 5 | 0.3 | Discharge from industrial chemical factories | Some people who use water containing 1,1,2-trichloroethane in excess of the MCL over many years may experience liver, kidney or immune system problems. |
| Trichloroethylene [TCE] (µg/L) | 0.005 | 1,000 | 5 | 1.7 | Discharge from metal degreasing sites and other factories | Some people who use water containing trichloroethylene in excess of the MCL over many years may experience liver problems and may have an increased risk of getting cancer. |
| Toluene (µg/L) | 0.15 | 1,000 | 150 | 150 | Discharge from petroleum and chemical factories; underground gas tank leaks | Some people who use water containing toluene in excess of the MCL over many years may experience nervous system, kidney, or liver problems. |
| Trichlorofluoromethane (µg/L) | 0.15 | 1,000 | 150 | 1300 | Discharge from industrial factories; degreasing solvent; propellant and refrigerant | Some people who use water containing trichlorofluoromethane in excess of the MCL over many years may experience liver problems. |
| 1,1,2-Trichloro-1,2,2-trifluoroethane (mg/L) | 1.2 | - | 1.2 | 4 | Discharge from metal degreasing sites and other factories; dry-cleaning solvent; refrigerant | Some people who use water containing 1,1,2-trichloro-1,2,2-trifluoroethane in excess of the MCL over many years may experience liver problems. |
| Vinyl Chloride (ng/L) | 0.0005 | 1,000,000 | 500 | 50 | Leaching from PVC piping; discharge from plastics factories; biodegradation byproduct of TCE and PCE groundwater contamination | Some people who use water containing vinyl chloride in excess of the MCL over many years may have an increased risk of getting cancer. |
| Xylenes (mg/L) | 1.750 | - | 1.750 | 1.8 | Discharge from petroleum and chemical factories; fuel solvent | Some people who use water containing xylenes in excess of the MCL over many years may experience nervous system damage. |

### Disinfection Byproducts, Disinfectant Residuals, and Disinfection Byproduct Precursors

| Contaminant (CCR units) | Traditional MCL or [MRDL] in mg/L | To convert for CCR, multiply by | MCL or [MRDL] in CCR units | PHG, (MCLG or MRDLG) | Major Sources in Drinking Water | Health Effects Language |
| --- | --- | --- | --- | --- | --- | --- |
| TTHMs [Total Trihalomethanes] (µg/L) | 0.080 | 1,000 | 80 | N/A | Byproduct of drinking water disinfection | Some people who drink water containing trihalomethanes in excess of the MCL over many years may experience liver, kidney, or central nervous system problems, and may have an increased risk of getting cancer. |
| HAA5 [Sum of 5 Haloacetic Acids] (µg/L) | 0.060 | 1,000 | 60 | N/A | Byproduct of drinking water disinfection | Some people who drink water containing haloacetic acids in excess of the MCL over many years may have an increased risk of getting cancer. |
| Bromate (µg/L) | 0.010 | 1,000 | 10 | 0.1 | Byproduct of drinking water disinfection | Some people who drink water containing bromate in excess of the MCL over many years may have an increased risk of getting cancer. |
| Chloramines (mg/L) | [MRDL = 4.0 (as Cl2)] | - | [MRDL = 4.0 (as Cl2)] | [MRDLG = 4 (as Cl2)] | Drinking water disinfectant added for treatment | Some people who use water containing chloramines well in excess of the MRDL could experience irritating effects to their eyes and nose. Some people who drink water containing chloramines well in excess of the MRDL could experience stomach discomfort or anemia. |
| Chlorine (mg/L) | [MRDL = 4.0 (as Cl2)] | - | [MRDL = 4.0 (as Cl2)] | [MRDLG = 4 (as Cl2)] | Drinking water disinfectant added for treatment | Some people who use water containing chlorine well in excess of the MRDL could experience irritating effects to their eyes and nose. Some people who drink water containing chlorine well in excess of the MRDL could experience stomach discomfort. |
| Chlorite (mg/L) | 1.0 | - | 1.0 | 0.05 | Byproduct of drinking water disinfection | Some infants and young children who drink water containing chlorite in excess of the MCL could experience nervous system effects. Similar effects may occur in fetuses of pregnant women who drink water containing chlorite in excess of the MCL. Some people may experience anemia. |
| Chlorine Dioxide (µg/L) | [MRDL = 0.8 (as ClO2)] | 1,000 | [MRDL = 800 (as ClO2)] | [MRDLG = 800 (as ClO2)] | Drinking water disinfectant added for treatment | Some infants and young children who drink water containing chlorine dioxide in excess of the MRDL could experience nervous system effects. Similar effects may occur in fetuses of pregnant women who drink water containing chlorine dioxide in excess of the MRDL. Some people may experience anemia. |
| Control of DBP Precursors (TOC) | TT | - | TT | N/A | Various natural and manmade sources | Total organic carbon (TOC) has no health effects. However, total organic carbon provides a medium for the formation of disinfection byproducts. These byproducts include trihalomethanes (THMs) and haloacetic acids (HAAs). Drinking water containing these byproducts in excess of the MCL may lead to adverse health effects, liver or kidney problems, or nervous system effects, and may lead to an increased risk of cancer. |

## APPENDIX B: Regulated Contaminants with Secondary Drinking Water Standards

Monitoring required by section 64449 of the California Code of Regulations, Title 22.

| Constituent | Secondary MCL (units) | To convert to CCR, multiply by | MCL in CCR units | Typical Source of Contaminant |
| --- | --- | --- | --- | --- |
| Aluminum | 0.2 mg/L | 1,000 | 200 µg/L | Erosion of natural deposits; residual from some surface water treatment processes |
| Color | 15 Units | - | 15 Units | Naturally-occurring organic materials |
| Copper | 1.0 mg/L | - | 1.0 mg/L | Internal corrosion of household plumbing systems; erosion of natural deposits; leaching from wood preservatives |
| Foaming Agents [MBAS] | 0.5 mg/L | 1,000 | 500 µg/L | Municipal and industrial waste discharges |
| Iron | 0.3 mg/L | 1,000 | 300 µg/L | Leaching from natural deposits; industrial wastes |
| Manganese | 0.05 mg/L | 1,000 | 50 µg/L | Leaching from natural deposits |
| Methyl-tert-butyl ether [MTBE] | 0.005 mg/L | 1,000 | 5 µg/L | Leaking underground storage tanks; discharge from petroleum and chemical factories |
| Odor---Threshold | 3 Units | - | 3 Units | Naturally-occurring organic materials |
| Silver | 0.1 mg/L | 1,000 | 100 µg/L | Industrial discharges |
| Thiobencarb | 0.001 mg/L | 1,000 | 1 µg/L | Runoff/leaching from rice herbicide |
| Turbidity | 5 Units | - | 5 Units | Soil runoff |
| Zinc | 5.0 mg/L | - | 5.0 mg/L | Runoff/leaching from natural deposits; industrial wastes |
| Total Dissolved Solids [TDS] | 1,000 mg/L | - | 1,000 mg/L | Runoff/leaching from natural deposits |
| Specific Conductance | 1,600 µS/cm | - | 1,600 µS/cm | Substances that form ions when in water; seawater influence |
| Chloride | 500 mg/L | - | 500 mg/L | Runoff/leaching from natural deposits; seawater influence |
| Sulfate | 500 mg/L | - | 500 mg/L | Runoff/leaching from natural deposits; industrial wastes |

Note: There are no PHGs, MCLGs, or mandatory standard health effects language for these constituents because secondary MCLs are set on the basis of aesthetic concerns.

## APPENDIX C: Monitored Contaminants with No MCLs

### Background

The 1996 Amendments to the SDWA required the U.S. EPA to establish criteria for a monitoring program for unregulated contaminants, and to publish, once every five years, a list of no more than 30 contaminants to be monitored by public water systems (PWS).

Section 64450 of the California Code of Regulations also required certain water systems to monitor a number of unregulated contaminants, with contaminant lists that were published or revised in 1990, 1996, 2000, and 2003. This section of the California Code of Regulations was repealed effective October 18, 2007. Water systems that continued to monitor for state unregulated contaminants are encouraged, but not required, to include the information regarding detected contaminants in the CCR.

Although Section 64450 of the California Code of Regulations was repealed, the State Water Board may request water systems to monitor for specific contaminants per HSC section 116375(b).

### Federal UCMR 1 (2001 – 2003 Monitoring)

The U.S. EPA published the first list of contaminants to monitor as part of the UCMR in September 1999. Contaminants were divided into two lists: Assessment Monitoring (List 1), and Screening Survey (List 2).

Assessment Monitoring of List 1 contaminants was conducted by large PWS serving more than 10,000 people and 800 representative small PWS serving 10,000 or fewer people. Assessment Monitoring was conducted by each PWS over a 12-month period between 2001 and 2003.

Screening Survey was conducted by a randomly selected set of 300 large and small PWSs for List 2 contaminants. Screening Survey for chemical contaminants was conducted in 2001 and 2002 for small and large PWS, respectively. Screening Survey for Aeromonas was conducted in 2003 for small and large PWS.

| UCMR 1 List 1 – Assessment Monitoring | UCMR 1 List 2 – Screening Survey |
| --- | --- |
| - 2,4-dinitrotoluene - 2,6-dinitrotoluene - Acetochlor - DCPA mono-acid degradate - DCPA di-acid degradate - 4,4’-DDE - EPTC - Molinate - MTBE - Nitrobenzene - Perchlorate - Terbacil | - 1,2-diphenylhydrazine - 2-methyl-phenol - 2,4-dichlorophenol - 2,4-dinitrophenol - 2,4,6-trichlorophenol - Aeromonas - Alachlor ESA - Diazinon - Disulfoton - Diuron - Fonofos - Linuron - Nitrobenzene - Prometon - Hexahydro-1,3,5-trinitro-1-3-5-triazine [RDX] - Terbufos |

### Federal UCMR 2 (2008 – 2010 Monitoring)

The U.S. EPA published the second list of contaminants to monitor as part of the UCMR in January 2007.

Assessment Monitoring was required of all PWS serving more than 10,000 people and 800 representative PWS serving 10,000 or fewer people for List 1 contaminants. Assessment Monitoring was required of each PWS during a 12-month period from January 2008 to December 2010.

Screening Survey was required of all PWS serving more than 100,000 people, 320 representative PWS serving 10,001 to 100,000 people, and 480 representative PWS serving 10,000 or fewer people for List 2 contaminants. Screening Survey was required of each PWS during a 12-month period from January 2008 to December 2010.

| UCMR 2 List 1 – Assessment Monitoring | UCMR 2 List 2 – Screening Survey |
| --- | --- |
| - Dimethoate - Terbufos sulfone - 2,2',4,4'-tetrabromodiphenyl ether - 2,2',4,4',5-pentabromodiphenyl ether - 2,2',4,4',5,5'-hexabromobiphenyl - 2,2',4,4',5,5'-hexabromodiphenyl ether - 2,2',4,4',6-pentabromodiphenyl ether - 1,3-dinitrobenzene - 2,4,6-trinitrotoluene (TNT) - Hexahydro-1,3,5-trinitro-1,3,5-trazine (RDX) | - Acetochlor ethane sulfonic acid - Acetochlor oxanilic acid - Alachlor ethane sulfonic acid - Alachlor oxanilic acid - Metolachlor ethane sulfonic acid - Metolachlor oxanilic acid - Acetochlor - Alachlor - Metolachlor - N-nitrosodiethylamine (NDEA) - N-nitrosodimethylamine (NDMA) - N-nitroso-di-n-butylamine (NDBA) - N-nitroso-di-n-propylamine (NDPA) - N-nitrosomethylethylamine (NMEA) - N-nitrosopyrrolidine (NPYR) |

### Federal UCMR 3 (2013 – 2015 Monitoring)

The third UCMR list of contaminants was published in May 2012.

Assessment Monitoring (List 1 Contaminants) was required of all PWS serving more than 10,000 people and 800 representative PWS serving 10,000 or fewer people. Assessment Monitoring was required of each PWS during a 12-month period from January 2013 to December 2015.

Screening Survey (List 2 Contaminants) was required of all PWS serving more than 100,000 people, 320 representative PWS serving 10,001 to 100,000 people, and 480 representative PWS serving 10,000 or fewer people. Screening Survey was required of each PWS during a 12-month period from January 2013 to December 2015.

Pre-screen Testing (List 3 Contaminants) was required from a selection of 800 representative PWS serving 1,000 or fewer people that do not disinfect. These PWS were selected because they have groundwater wells that were located in areas of karst or fractured bedrock. Monitored lasted 12 months between January 2013 and December 2015.

| UCMR 3 List 1 – Assessment Monitoring | UCMR 3 List 2 – Screening Survey | UCMR 3 List 3 – Pre-Screen Testing |
| --- | --- | --- |
| - 1,2,3-trichloropropane - 1,3-butadiene - Chloromethane (methyl chloride) - 1,2-dichloroethane - Bromomethane (methyl bromide) - Chlorodifluoromethane (HCFC-22) - Bromochloromethane (halon 1011) - 1,4-dioxane - Vanadium - Molybdenum - Cobalt - Strontium - Chromium (total) - Chromium-6 - Chlorate - Perfluorooctanesulfonate acid (PFOS) - Perfluorooctanoic acid (PFOA) - Perfluorononanoic acid (PFNA) - Perfluorohexanesulfonic acid (PFHxS) - Perfluoroheptanoic acid (PFHpA) - Perfluorobutanesulfonic acid (PFBS) | - 17-β-estradiol - 17-α-ethynylestradiol (ethinyl estradiol) - 16-α-hydroxyestradiol (estriol) - Equilin - Estrone - Testosterone - 4-anderostene-3,17-dione | - Enteroviruses - Noroviruses |

### Federal UCMR 4 (2018 – 2020 Monitoring)

The fourth list of contaminants to monitor as part of the UCMR was published by the U.S. EPA in December 2016.

PWSs are required to monitor for 10 cyanotoxins at the entry point to the distribution system during a 4-consecutive month period from March 2018 through November 2020, according to the table below. PWSs are also required to monitor for 20 additional chemical contaminants and indicators during a 12-month period from January 2018 through December 2020. The sampling site for these additional chemicals is the entry point to the distribution system, except for HAAs that need to be monitored at the Stage 2 D/DBPR sampling sites. The two indicators, i.e., TOC and bromide, need to be monitored at source water intakes.

| System Size (Population Served) | 10 Cyanotoxins | 20 Chemicals |
| --- | --- | --- |
| Small Systems (25 – 10,000) | 800 randomly selected surface water or ground water under the direct influence of surface water (GWUDI) systems | A different group of 800 randomly selected surface water systems, GWUDI and groundwater systems |
| Large Systems (10,001 or more) | All surface water and GWUDI systems | All surface water, groundwater and GWUDI systems |

The 10 cyanotoxins and 20 additional chemical contaminants and indicators are listed in the table below.

#### UCMR 4 Chemical Contaminants and Indicators

| Cyanotoxins | Minimum Reporting Level |
| --- | --- |
| Total Microcystin | 0.3 µg/L |
| Microcystin-LA | 0.008 µg/L |
| Microcystin-LF | 0.006 µg/L |
| Microcystin-LR | 0.02 µg/L |
| Microcystin-LY | 0.009 µg/L |
| Microcystin-RR | 0.006 µg/L |
| Microcystin-YR | 0.02 µg/L |
| Nodularin | 0.005 µg/L |
| Anatoxin-a | 0.03 µg/L |
| Cylindrospermopsin | 0.09 µg/L |

| Additional Chemicals | Minimum Reporting Level |
| --- | --- |
| Germanium | 0.3 µg/L |
| Manganese | 0.4 µg/L |
| Alpha-hexachlorocyclohexane | 0.01 µg/L |
| Chlorpyrifos | 0.03 µg/L |
| Dimethipin | 0.2 µg/L |
| Ethoprop | 0.03 µg/L |
| Oxyfluorfen | 0.05 µg/L |
| Profenofos | 0.3 µg/L |
| Tebuconazole | 0.2 µg/L |
| Total Permethrin (cis- & trans-) | 0.04 µg/L |
| Tribufos | 0.07 µg/L |
| HAA5 | N/A |
| HAA6Br1 | N/A |
| HAA92 | N/A |
| 1-butanol | 2.0 µg/L |
| 2-methoxyethanol | 0.4 µg/L |
| 2-propen-1-ol | 0.5 µg/L |
| butylated hydroxyanisole | 0.03 µg/L |
| o-toluidine | 0.007 µg/L |
| quinoline | 0.02 µg/L |
| Total Organic Carbon (TOC) | N/A |
| Bromide | N/A |

1 HAA6Br: Bromochloroacetic acid, bromodichloroacetic acid, dibromoacetic acid, dibromochloroacetic acid, monobromoacetic acid, and tribromoacetic acid.

2 HAA9: Bromochloroacetic acid, bromodichloroacetic acid, chlorodibromoacetic acid, dibromoacetic acid, dichloroacetic acid, monobromoacetic acid, monochloroacetic acid, tribromoacetic acid, and trichloroacetic acid.

### Federal UCMR 5 (2023 – 2025 Monitoring)

The Fifth Unregulated Contaminant Monitoring Rule (UCMR5) was published by the U.S. EPA in December 2021. As part of this rule, public water systems (PWS) are required to monitor for 29 PFAS and lithium. The table below shows each of the chemicals included in monitoring and the associated minimum reporting level.

#### UCMR 5 Chemicals and Minimum Reporting Levels

| Chemical | Minimum Reporting Level (µg/L) |
| --- | --- |
| 11-chloroeicosafluoro-3-oxaundecane-1-sulfonic acid (11Cl-PF3OUdS) | 0.005 |
| 1H,1H, 2H, 2H-perfluorodecane sulfonic acid (8:2FTS) | 0.005 |
| 1H,1H, 2H, 2H-perfluorohexane sulfonic acid (4:2FTS) | 0.003 |
| 1H,1H, 2H, 2H-perfluorooctane sulfonic acid (6:2FTS) | 0.005 |
| 4,8-dioxa-3H-perfluorononanoic acid (ADONA) | 0.003 |
| 9-chlorohexadecafluoro-3-oxanonane-1-sulfonic acid (9Cl-PF3ONS) | 0.002 |
| hexafluoropropylene oxide dimer acid (HFPO-DA)(GenX) | 0.005 |
| nonafluoro-3,6-dioxaheptanoic acid (NFDHA) | 0.02 |
| perfluoro (2-ethoxyethane) sulfonic acid (PFEESA) | 0.003 |
| perfluoro-3-methoxypropanoic acid (PFMPA) | 0.004 |
| perfluoro-4-methoxybutanoic acid (PFMBA) | 0.003 |
| perfluorobutanesulfonic acid (PFBS) | 0.003 |
| perfluorobutanoic acid (PFBA) | 0.005 |
| perfluorodecanoic acid (PFDA) | 0.003 |
| perfluorododecanoic acid (PFDoA) | 0.003 |
| perfluoroheptanesulfonic acid (PFHpS) | 0.003 |
| perfluoroheptanoic acid (PFHpA) | 0.003 |
| perfluorohexanesulfonic acid (PFHxS) | 0.003 |
| perfluorohexanoic acid (PFHxA) | 0.003 |
| perfluorononanoic acid (PFNA) | 0.004 |
| perfluorooctanesulfonic acid (PFOS) | 0.004 |
| perfluorooctanoic acid (PFOA) | 0.004 |
| perfluoropentanesulfonic acid (PFPeS) | 0.004 |
| perfluoropentanoic acid (PFPeA) | 0.003 |
| perfluoroundecanoic acid (PFUnA) | 0.002 |
| N-ethyl perfluorooctanesulfonamidoacetic acid (NEtFOSAA) | 0.005 |
| N-methyl perfluorooctanesulfonamidoacetic acid (NMeFOSAA) | 0.006 |
| perfluorotetradecanoic acid (PFTA) | 0.008 |
| perfluorotridecanoic acid (PFTrDA) | 0.007 |
| lithium | 9 |

For more information, please visit https://www.epa.gov/dwucmr/fifth-unregulated-contaminant-monitoring-rule.

### Reporting

U.S. EPA is essentially silent on the issue of reporting federal UCMR contaminants beyond the previous calendar year’s detections, other than to say it is not required and that data older than five years need not be reported. As a result, the State Water Board recommends systems to report data for five years from the date of the last sampling.

## APPENDIX D: State Contaminants with Notification Levels

Inclusion of the Notification Level (NL) and health effects language for contaminant concentrations detected above the NL is recommended but not required.

| Chemical | Notification Level | Health Effects Language (Optional) |
| --- | --- | --- |
| Boron | 1 mg/L | Boron exposures resulted in decreased fetal weight (developmental effects) in newborn rats. |
| n-Butylbenzene | 260 µg/L | Exposures to cumene (isopropylbenzene), a surrogate for n-, sec-, and tert-butylbenzene, resulted in increased kidney weight in rats. |
| sec-Butylbenzene | 260 µg/L | Exposures to cumene (isopropylbenzene), a surrogate for n-, sec-, and tert-butylbenzene, resulted in increased kidney weight in rats. |
| tert-Butylbenzene | 260 µg/L | Exposures to cumene (isopropylbenzene), a surrogate for n-, sec-, and tert-butylbenzene, resulted in increased kidney weight in rats. |
| Carbon Disulfide | 160 µg/L | Carbon disulfide exposures resulted in decreased motor conduction velocity in people. |
| Chlorate | 800 µg/L | Animal studies demonstrated that chlorate exposure in rats caused adverse effects to the pituitary and thyroid glands. |
| 2-Chlorotoluene | 140 µg/L | 2-Chlorotoluene exposures resulted in decrease in body weight gain in rats. |
| 4-Chlorotoluene | 140 µg/L | 4-Chlorotoluene is expected to have health effects similar to those of 2-chlorotoluene. |
| Diazinon | 1.2 µg/L | Diazinon exposures may result in neurotoxic effects. |
| Dichlodifluoromethane [Freon 12] | 1 mg/L | Dichlorodifluoromethane exposures resulted in reduced body weight in rats. |
| 1,4-Dioxane | 1 µg/L | 1,4-Dioxane exposures resulted in cancer, based on studies in laboratory animals. |
| Ethylene Glycol | 14 mg/L | Ethylene glycol exposures resulted in kidney toxicity in rats. |
| Formaldehyde | 100 µg/L | Formaldehyde exposures resulted in reduced weight gain and histopathology in rats. |
| Octahydro-1,3,5,7-tetranitro-1,3,5,7-tetrazocine [HMX] | 350 µg/L | Octahydro-1,3,5,7-tetranitro-1,3,5,7-tetrazocine exposures resulted in liver lesions in rats. |
| Isopropylbenzene | 770 µg/L | Isopropylbenzene exposures resulted in increased kidney weight in rats. |
| Manganese | 50 µg/L | High levels of manganese exposure have been shown to result in neurological impacts; formula-fed infants may be particularly at risk. |
| Methyl Isobutyl Ketone [MIBK] | 120 µg/L | Methyl isobutyl ketone exposures resulted in increased kidney and liver weight, and kidney pathology in rats. |
| Naphthalene | 17 µg/L | Naphthalene exposures resulted in decreased body weight in rats. |
| N-Nitrosodiethylamine [NDEA] | 10 ng/L | N-nitrosodiethylamine exposures resulted in cancer in a variety of laboratory animals. |
| N-Nitrosodimethylamine [NDMA] | 10 ng/L | N-nitrosodimethylamine exposures resulted in cancer in a variety of laboratory animals. |
| N-Nitrosodi-n-propylamine [NDPA] | 10 ng/L | N-nitrosodi-n-propylamine exposures resulted in cancer in a variety of laboratory animals. |
| Perfluorobutane sulfonic acid [PFBS] | 500 ng/L | Perfluorobutane sulfonic acid exposures resulted in decreased thyroid hormone in pregnant female mice. |
| Perfluorohexanoic Acid [PFHxA] | 1.0 µg/L | Perfluorohexanoic acid has been shown to interfere with levels of thyroid hormones. |
| Perfluorohexane Sulfonic Acid [PFHxS] | 3.0 ng/L | Perfluorohexane sulfonic acid exposures resulted in decreased total thyroid hormone in male rats. |
| Perfluorooctanoic Acid [PFOA] | 4.0 ng/L** | Perfluorooctanoic acid exposures resulted in increased liver weight and cancer in laboratory animals. |
| Perfluorooctanesulfonic Acid [PFOS] | 4.0 ng/L** | Perfluorooctanesulfonic acid exposures resulted in immune suppression and cancer in laboratory animals. |
| Propachlor | 90 µg/L | Propachlor exposures resulted in decrease in weight gain, decrease in food intake, and relative liver weight increase in rats. |
| n-Propylbenzene | 260 µg/L | Exposures to cumene (isopropylene), a surrogate for n-propylbenzene, resulted in increased kidney weight in rats. |
| Hexahydro-1,3,5-trinitro-1-3-5-triazine [RDX] | 300 ng/L | Hexahydro-1,3,5-trinitro-1-3-5-triazine exposures resulted in liver carcinomas and adenomas in female mice. |
| Tertiary Butyl Alcohol [TBA] | 12 µg/L | Tert-butyl alcohol exposures resulted in cancer in laboratory animals. |
| 1,2,4-Trimethylbenzene | 330 µg/L | 1,2,4-Trimethylbenzene exposures resulted in increased serum phosphorus levels in rats. |
| 1,3,5-Trimethylbenzene | 330 µg/L | 1,3,5-Trimethylbenzene exposures resulted in increased serum phosphorus levels in rats. |
| 2,4,6-Trinitrotoluene [TNT] | 1 µg/L | 2,4,6-Trinitrotoluene exposures resulted in urinary bladder transitional cell papillomas and squamous cell carcinomas in female rats. |
| Vanadium | 50 µg/L | Vanadium exposures resulted in developmental and reproductive effects in rats. |

## APPENDIX E: California’s Detection Limits for Purposes of Reporting (DLRs)

Most contaminants with primary MCLs have DLRs that are in the California Code of Regulations. A list of MCLs, DLRs, and PHGs may be found via the link labeled “MCLs, DLRs, and PHGs - (PDF)” on the State Water Board website, which is available at https://www.waterboards.ca.gov/drinking_water/certlic/drinkingwater/MCLsandPHGs.html.

Many of the DLRs are for contaminants that you do not have to report in your CCR; however, the State Water Board encourages you to report any of these that you find, particularly if there is: (1) a proposed regulation for the detected chemical, or (2) health guidance available (U.S. EPA health advisory or State Water Board NL).

If you are uncertain about the inclusion of certain data, talk to your primacy agency. If you cannot find a contaminant listed below and your laboratory analysis provides a detected value for that contaminant, the State Water Board recommends that you report it in your CCR. If you are uncertain, always provide too much data rather than too little.

## APPENDIX F: Reporting Monitoring Data

This appendix provides examples of monitoring data and instructions on how to report certain detects in the Consumer Confidence Report (CCR).

Note: Non-detected contaminants should not be included in a table of detected contaminants. However, the initials “ND” (no detection) may be used in a table of detected contaminants as a placeholder, for example, when presenting quarterly data, an average, or a data range for a detected contaminant. Please note any requirements when non-detection should be reported as a zero (“0”). See below for examples.

### Example: Reporting for one sampling site and one sampling date

[Cal. Code Regs., Title 22, § 64481(d)(2)(D)1.A.]

Example Data - Barium Monitoring

| Barium MCL | 1 mg/L |
| --- | --- |
| MCL in CCR units | 1 mg/L |
| March 2026 Result | 0.003 mg/L |

Example CCR Table Excerpt

| Contaminant (CCR units) | MCL | PHG | Average | Range | Sample Date | Violation | Typical Source |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Barium (mg/L) | 1 | 2 | 0.003 | N/A | 2026 | No | Discharges of oil drilling wastes and from metal refineries; erosion of natural deposits |

Note: Monitoring frequency is once every 3 years, therefore the water system will report this same result each CCR year until the next sample is taken.

### Example: Reporting for one sampling site and multiple sampling dates

[Cal. Code Regs., Title 22, § 64481(d)(2)(D)1.A.]

Example Data – Xylenes Monitoring

| Xylenes MCL | 1.750 mg/L |
| --- | --- |
| MCL in CCR units | 1.750 mg/L |

| Quarter | 1st Qtr | 2nd Qtr | 3rd Qtr | 4th Qtr | Average |
| --- | --- | --- | --- | --- | --- |
| 2026 Results for Source Reported Individually in CCR Table (mg/L) | 1 | 1 | 2 | ND | 1 |

Example CCR Table Excerpt

| Contaminant (CCR units) | MCL | PHG | Average | Range | Sample Date | Violation | Typical Source |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Xylenes (mg/L) | 1.750 | 1.8 | 1 | ND - 2 | 2022 | No | Discharge from petroleum and chemical factories; fuel solvent |

### Example: Reporting for multiple sampling sites and one sampling date

[Cal. Code Regs., Title 22, § 64481(d)(2)(D)1.B.]

Example Data – Barium Monitoring

| Barium MCL | 1 mg/L |
| --- | --- |
| MCL in CCR units | 1 mg/L |

| Source | Well 1 | Well 2 | Well 3 | Average |
| --- | --- | --- | --- | --- |
| March 2026 Results (mg/L) | 0.60 | 0.46 | ND | 0.35 |

Example CCR Table Excerpt

| Contaminant (CCR units) | MCL | PHG | Average | Range | Sample Date | Violation | Typical Source |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Barium (mg/L) | 1 | 2 | 0.35 | ND – 0.60 | 2026 | No | Discharges of oil drilling wastes and from metal refineries; erosion of natural deposits |

Note: Monitoring frequency is once every 3 years, therefore the water system will report these same results each CCR year until the next sample is taken.

If reporting a flow-weighted average:

If these sources enter the distribution system at the same point, a flow-weighted average may be reported instead of the average computed as follows:

Assuming Wells 1, 2, and 3 contributed 20, 40 and 40 percent, respectively.

Weighted average = 0.2(0.60 mg/L) + 0.4(0.46 mg/L) + 0.4(ND mg/L) = 0.30 mg/L.

Example CCR Table Excerpt

| Contaminant (CCR units) | MCL | PHG | Average | Range | Sample Date | Violation | Typical Source |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Barium (mg/L) | 1 | 2 | 0.30 | ND – 0.60 | 2026 | No | Discharges of oil drilling wastes and from metal refineries; erosion of natural deposits |

Note: Monitoring frequency is once every 3 years, therefore the water system will report these same results each CCR year until the next sample is taken.

### Example: Reporting for multiple sampling sites and multiple sampling dates (compliance: four quarter or six-month average)

[Cal. Code Regs., Title 22, § 64481(d)(2)(D)1.C.]

Example Data – Xylenes Monitoring

| Xylenes MCL | 1.750 mg/L |
| --- | --- |
| MCL in CCR units | 1.750 mg/L |

2026 Results (mg/L)

| Source | 1st Qtr | 2nd Qtr | 3rd Qtr | 4th Qtr | Average |
| --- | --- | --- | --- | --- | --- |
| Well 1 | 1 | 1 | 2 | ND | 1 |
| Well 2 | Not sampled | Not sampled | Not sampled | 2 | 2 |
| Well 3 | Not sampled | 1 | 2 | ND | 1 |
| All wells | NA | NA | NA | NA | 1.3 |

Example CCR Table Excerpt

| Contaminant (CCR units) | MCL | PHG | Average | Range | Sample Date | Violation | Typical Source |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Xylenes (mg/L) | 1.750 | 1.8 | 1.3 | ND - 2 | 2020 | No | Discharge from petroleum and chemical factories; fuel solvent |

Note: For “Average”, use the average of the individual sampling point averages.

### Example: Reporting of multiple sampling sites and multiple sampling dates (compliance: RAA on individual source basis)

[Cal. Code Regs., Title 22, § 64481(d)(2)(D)2.A.]

Example Data – Dalapon Monitoring

| Dalapon MCL | 0.2 mg/L |
| --- | --- |
| MCL in CCR units | 200 µg/L |

2026 Results (µg/L)

| Source | 1st Qtr Sample Result | 1st Qtr Running Average1 | 2nd Qtr Sample Result | 2nd Qtr Running Average1 | 3rd Qtr Sample Result | 3rd Qtr Running Average1 | 4th Qtr Sample Result | 4th Qtr Running Average1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Well 1 | 74 | 42 | 60 | 47 | 28 | 50 | 43 | 52 |
| Well 2 | 36 | 26 | 12 | 21 | 6 | 17 | 9 | 16 |

1 RAAs are based on data from previous quarters not reported on this table.

Example CCR Table Excerpt

| Contaminant (CCR units) | MCL | PHG | Average | Range | Sample Date | Violation | Typical Source |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Dalapon (µg/L) | 200 | 790 | 52 | 6 - 74 | 2026 | No | Runoff from herbicide used on rights-of-way, and crops and landscape maintenance |

Note: For “Average”, use the highest of the RAAs for the year. For “Range”, use the minimum and maximum of the sample results for all locations, all dates.

### Example: Reporting of multiple sampling sites and multiple sampling dates (compliance: LRAA – TTHM and HAA5)

[Cal. Code Regs., Title 22, § 64481(d)(2)(D).2.B.]

#### For a System with No LRAA Exceedance:

Example Data – TTHM Monitoring (state Stage 2 D/DBPR)

| TTHM MCL | 0.080 mg/L |
| --- | --- |
| MCL in CCR units | 80 µg/L |

2026 TTHM Results (µg/L)

| Location | 1st Qtr | 2nd Qtr | 3rd Qtr | 4th Qtr |
| --- | --- | --- | --- | --- |
| Site 1 | 45 | 60 | 125 | 70 |
| Site 1 LRAA1 | 44 | 59 | 74 | 75 |
| Site 2 | 40 | 55 | 115 | 60 |
| Site 2 LRAA1 | 39 | 53 | 67 | 68 |
| Site 3 | 45 | 60 | 105 | 70 |
| Site 3 LRAA1 | 44 | 61 | 69 | 70 |
| Site 4 | 50 | 65 | 120 | 75 |
| Site 4 LRAA1 | 49 | 63 | 74 | 78 |

1 LRAA for Quarters 1 to 3 are based on results from previous quarters not reported on this table.

Example CCR Table Excerpt

| Contaminant (CCR units) | MCL | PHG (or MCLG) | Average | Range | Sample Date | Violation | Typical Source |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TTHM (µg/L) | 80 | N/A | 78 | 40 - 125 | 2026 | No | Byproduct of drinking water disinfection |

Note: For “Average”, use the highest of the LRAAs for the year. For “Range”, use the minimum and maximum of the sample results for all locations, all dates.

#### For a System with an LRAA Exceedance:

Example Data – TTHM Monitoring (state Stage 2 D/DBPR)

| TTHM MCL | 0.080 mg/L |
| --- | --- |
| MCL in CCR units | 80 µg/L |

2026 TTHM Results (µg/L)

| Location | 1st Qtr | 2nd Qtr | 3rd Qtr | 4th Qtr |
| --- | --- | --- | --- | --- |
| Site 1 | 45 | 60 | 125 | 100 |
| Site 1 LRAA1 | 44 | 59 | 74 | 83 |
| Site 2 | 40 | 55 | 115 | 60 |
| Site 2 LRAA1 | 39 | 53 | 67 | 68 |
| Site 3 | 45 | 60 | 105 | 70 |
| Site 3 LRAA1 | 44 | 61 | 69 | 70 |
| Site 4 | 50 | 65 | 120 | 75 |
| Site 4 LRAA1 | 49 | 63 | 74 | 78 |

1 LRAAs for Quarters 1 to 3 are based on results from previous quarters not reported on this table.

Example CCR Table Excerpt

| Contaminant (CCR units) | MCL | PHG (or MCLG) | Average | Range | Sample Date | Violation | Typical Source |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TTHM (µg/L) | 80 | N/A | 83 | 40 - 125 | 2026 | Yes1 | Byproduct of drinking water disinfection |

1 See Checklist Item 4, subitem 8 for instructions on how to highlight the violation in the table and the additional information to include near, but not in the table.

Note: For “Average”, use the highest of the LRAAs for the year. For “Range”, use the minimum and maximum of the sample results for all locations, all dates in this year.

### Example: Reporting of turbidity results

[Cal. Code Regs., Title 22, § 64481(d)(2)(E)]

When reporting turbidity as an indicator of filtration performance, systems must report the highest single measurement and the lowest monthly percentage of samples meeting the requirements specified for that technology, and an explanation of the reasons for monitoring turbidity (Checklist Item 4). In this situation, you may want to report the data in 2 rows of your table.

#### For conventional or direct filtration:

This applies to systems subject to the state Interim Enhanced Surface Water Treatment Rule or Long Term 1 Enhanced Surface Water Treatment Rule.

Example CCR Table Excerpt

| Contaminant | MCL | PHG | Level Found | Range | Sample Date | Violation | Typical Source |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Turbidity | TT = 1 NTU | N/A | 0.7 NTU | N/A | 2026 | No | Soil runoff |
| Turbidity | TT = 95% of samples ≤0.3 NTU | N/A | 93% | N/A | 2026 | No | Soil runoff |

#### For diatomaceous earth filtration:

Example CCR Table Excerpt

| Contaminant | MCL | PHG | Level Found | Range | Sample Date | Violation | Typical Source |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Turbidity | TT = 5.0 NTU | N/A | 1 NTU | N/A | 2026 | No | Soil runoff |
| Turbidity | TT = 95% of samples ≤0.5 NTU | N/A | 93% | N/A | 2026 | No | Soil runoff |

#### For slow sand filtration:

Example CCR Table Excerpt

| Contaminant | MCL | PHG | Level Found | Range | Sample Date | Violation | Typical Source |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Turbidity | TT = 5.0 NTU | N/A | 1 NTU | N/A | 2026 | No | Soil runoff |
| Turbidity | TT = 95% of samples ≤1.0 NTU | N/A | 93% | N/A | 2026 | No | Soil runoff |

#### For alternative filtration technology:

Example CCR Table Excerpt

| Contaminant | MCL | PHG | Level Found | Range | Sample Date | Violation | Typical Source |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Turbidity | TT = * | N/A | [INSERT LEVEL] | N/A | 2026 | [INSERT VIOLATION] | Soil runoff |
| Turbidity | TT = * | N/A | [INSERT LEVEL] | N/A | 2026 | [INSERT VIOLATION] | Soil runoff |

* Refer to turbidity limits established by the State Water Board.

### Example: Reporting of lead results

[Cal. Code Regs., Title 22, § 64481(d)(2)(F)]

Example Data – Lead Monitoring

| Lead Action Level (90th percentile) | 0.015 mg/L |
| --- | --- |
| Lead DLR | 5 µg/L |
| AL in CCR units | 15 µg/L |

| July 2026 Lead Results (µg/L) | Site 1 | Site 2 | Site 3 | Site 4 | Site 5 | Site 6 | Site 7 | Site 8 | Site 9 | Site 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lab Reported Results | ND | ND | 8 | 12 | 9 | 5 | ND | <5 | 6 | 10 |
| Results Converted per section 64678(c) | 0 | 0 | 8 | 12 | 9 | 5 | 0 | 0 | 6 | 10 |

Note: If the laboratory reports a finding of “ND” or “< (numerical value of DLR)”, replace the finding with a “zero” (0) per Cal. Code Regs, Title 22, § 64678(c).

To calculate the 90th percentile: The results of all samples taken during a monitoring period shall be placed in ascending order from the sample with the lowest concentration to the sample with the highest concentration. Each sample result shall be assigned a number starting with the number 1 for the lowest value. The number of samples taken during the monitoring period shall be multiplied by 0.9. The contaminant concentration in the numbered sample yielded by this calculation is the 90th percentile.

Example Lead Results in Order

| Order Number | Order 1 | Order 2 | Order 3 | Order 4 | Order 5 | Order 6 | Order 7 | Order 8 | Order 9 | Order 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| July 2026 Lead Results (µg/L) | 0 | 0 | 0 | 0 | 5 | 6 | 8 | 9 | 10 | 12 |

10 samples x 0.9 = 9; therefore, the ninth value is the 90th percentile value (10 µg/L).

Report in CCR Table: 90th percentile = 10, number of sites sampled = 10, and number of sites above AL (15 µg/L) = 0.

For small systems collecting 5 samples, if the average of the two highest samples is below the reporting level of 5 µg/L, then there is no need to report any detection for lead. If there is only one sample with a detection, then the detected level is divided by 2 and if the result is at or above 5 µg/L, then it should be reported on the CCR.

Public education language is presented in Checklist Item 5.

Water quality parameter data that you collect in association with the LCR should not be included in the CCR.

Example CCR Table Excerpt

| Contaminant (CCR units) | MCL | PHG | 90th Percentile | Results | Range | Sample Date | Violation | Typical Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lead (µg/L) | AL = 15 | 0.2 | 10 | 10 sites sampled; 0 sites over AL | 0 to 12 | 2026 | No | Internal corrosion of household water plumbing systems; discharges from industrial manufacturers; erosion of natural deposits |

### Example: Reporting of fluoride results

[Cal. Code Regs., Title 22, § 64481(d)(2)(D)1.A. and B]

The table below assumes one sampling site and one sampling date.

Example CCR Table Excerpt

| Contaminant (CCR units) | MCL | PHG | Average | Range | Sample Date | Violation | Typical Source |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Fluoride (naturally occurring) (mg/L)1 | 2.0 | 1 | 0.3 | [INSERT RANGE] | 2026 | N/A | Erosion of natural deposits; water additive that promotes strong teeth; discharge from fertilizer and aluminum factories |

1 (Sample wording if fluoride added to the water): Our water system treats your water by adding fluoride to the naturally occurring level to help prevent dental caries in consumers. State regulations require the fluoride levels in the treated water be maintained within a range of [list control range] mg/L with an optimum dose of [list value] mg/L. Our monitoring showed that the fluoride levels in the treated water ranged from [INSERT RANGE] with an average of [list value] mg/L. Information about fluoridation, oral health, and current issues is available from https://www.waterboards.ca.gov/drinking_water/certlic/drinkingwater/Fluoridation.html.

### Example: Reporting of radioactivity results

[Cal. Code Regs., Title 22, § 64481(c)(1)]

Gross alpha monitoring results are used for two purposes: To determine compliance with the gross alpha MCL and to screen for radium and uranium. In both cases, an average of four quarterly samples is used unless the samples have been composited.

Determining MCL compliance: Counting errors and minimum detectable activity (MDA) are not included in the averages of gross alpha, uranium, or radium data used to determine compliance with the MCLs. Therefore, they are not included in the data reported in the CCR.

Screening to determine if radium and uranium testing is necessary: When the gross alpha data are averaged to determine whether radium or uranium testing should be conducted, counting errors are added in and the MDA is substituted in for any zero result. Confusion about radioactivity data reporting for the CCR has resulted from the way the average is calculated for screening purposes, but this approach is not appropriate for CCR data reporting.

### Example: Reporting of sodium and hardness results

[Cal. Code Regs. § 64481(c)(4)]

Although sodium and hardness do not have MCLs, they are of interest to many consumers who are concerned about sodium intake and may believe that the hardness of the water could affect their health. Therefore, monitoring is required and detections should be included in the table(s) along with the other data on water quality. Since there are no MCLs/PHGs/MCLGs, just indicate that in the table in some way (e.g., none, N/A).

No “typical source” is required, though a system may wish to include information such as:

“Hardness” is the sum of polyvalent cations present in the water, generally magnesium and calcium. The cations are usually naturally occurring.

“Sodium” refers to the salt present in the water and is generally naturally occurring.

### Example: Reporting of Stage 1 D/DBPR TOC treatment technique violations (surface water treatment plants with conventional treatment or precipitative softening)

[Cal. Code Regs., Title 22, § section 64481(d)(3)]

If any of the following apply, you must report a TT violation for enhanced coagulation or enhanced softening (if applicable):

- Alternative compliance criteria for enhanced coagulation or enhanced softening cannot be met.

- Quarterly TOC monitoring does not demonstrate the percentage removal of TOC (demonstrated in the table below).

- A system does not obtain state approval for alternate minimum TOC removal (Step 2) requirements.

Example Data – For a conventional surface water treatment system with source water TOC between 2 – 4 mg/L and with a source water alkalinity between 0 – 50 mg/L.

Example CCR Table Excerpt

| TT Violation | Explanation | Length | Steps Taken to Correct the Violation | Health Effect Language |
| --- | --- | --- | --- | --- |
| Failure to remove required amount of total organic carbon (TOC) | On [date], we collected samples for TOC before and after our treatment process to determine the percentage of TOC we were removing. Results showed that we were removing 25 percent of the TOC. We are required to remove 35 percent of the TOC. | 1 month | We examined our treatment processes to see if we could improve our removal of TOC. We made some adjustments to our process on [date]. Samples collected after that time show that we are able to achieve 35 percent removal. | Total organic carbon (TOC) has no health effects. However, TOC provides a medium for the formation of disinfection byproducts. These byproducts include trihalomethanes (THMs) and haloacetic acids (HAAs). Drinking water containing these byproducts in excess of the MCL may lead to adverse health effects, liver, or kidney problems, or nervous system effects, and may lead to an increased risk of getting cancer. |

### Example: Reporting of LT2ESWTR treatment technique violations

For violation of TTs under the LT2ESWTR, the system must provide an explanation of the violation, an indication of the length of the violation, information on steps taken to correct the violation, and health effects language. Because there are no standard health effects language provided for these TTs, the system would have to write language specific to the violation. You can use the health effect language for contaminants as an example or template.

Example CCR Table Excerpt

| TT Violation | Explanation | Length | Steps Taken to Correct the Violation | Health Effect Language |
| --- | --- | --- | --- | --- |
| Uncovered and Untreated Finished Water Reservoir | The Alma finished water reservoir is uncovered and the discharge is not treated. We were required to address this situation by [INSERT DATE]. | 17 months | We have hired an engineering firm to design a cover for the tank. We intend to have the tank covered by [INSERT DATE]. | Inadequately protected water may contain disease-causing organisms. These organisms can cause symptoms such as diarrhea, nausea, cramps, and associated headaches. |
| Determine and Report Bin Classification | After conducting our source water monitoring for Cryptosporidium, we were required to determine and report our Bin Classification by [INSERT DATE]. | 1 month | We have since determined our bin classification and reported this to the State Water Resources Control Board. | Inadequately treated water may contain disease-causing organisms. These organisms can cause symptoms such as diarrhea, nausea, cramps, and associated headaches. |
| Provide or Install an Additional Level of Treatment | Based on our bin classification, we were required to provide or install an additional level of treatment by [INSERT DATE]. | 6 months | We hired an engineering firm to prepare a preliminary engineering report. The report listed treatment alternatives. We selected one of the alternatives and are in the process of constructing it. We anticipate that it will be completed by [INSERT DATE]. | Inadequately treated water may contain disease-causing organisms. These organisms can cause symptoms such as diarrhea, nausea, cramps, and associated headaches. |

### Example: Reporting of GWR special notice for fecal indicator-positive groundwater source sample

#### Example Data – E. coli

This system was triggered to conduct source water monitoring after a RTCR positive sample in December 2026. In this example, both the distribution and the source samples were positive for E. coli. The system took five additional source samples and one was positive. Below is an example of reporting for both the RTCR violation and the GWR special notice.

The required special notice language for fecal indicator-positive samples must be provided in the CCR. For this example, we have included it as footnote to the table.

Example CCR Table Excerpt – E. coli

| Contaminant | MCL | MCLG | Your Water | Range | Sample Date | Violation | Typical Source |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E. coli (in the distribution system) | 0 | 0 | 1 positive sample | ND - 1 | 2026 | Yes1 | Human or animal fecal waste |
| E. coli (at the groundwater source)2 | 0 | 0 | 2 positive samples | ND - 1 | 2026 | No | Human or animal fecal waste |

1 We were notified on December 9, 2026, of an E. coli positive sample in the distribution system. You may remember receiving public notification of this violation on December 10. For reasons discussed in the next paragraph, we took Well 1 off-line on December 11. The duration of the violation was two days. We are addressing this contaminated well as discussed below.

2 On December 20, 2026, we sampled the sources (Well 1 and Well 2) for the fecal-indicator, E. coli. We were notified on December 11 that Well 1 tested positive for E. coli. On December 12, we took five additional samples and were notified on December 13 that two of the five samples were positive for E. coli. We immediately took Well 1 off-line at that time. Our system is in contact with the State Water Resources Control Board, and we have a State Water Board-approved plan to abandon this well and replace it with a new well. We will have the new well completed by July 5, 2027, and the old well will be abandoned by July 15, 2027. As an interim measure, we have moved to only utilizing this well as an emergency source and have not had to utilize it since the sampling revealed the contamination.

Health Effects: E. coli are bacteria whose presence indicates that the water may be contaminated with human or animal wastes. Human pathogens in these wastes can cause short-term effects, such as diarrhea, cramps, nausea, headaches, or other symptoms. They may pose a greater health risk for infants, young children, some of the elderly, and people with severely-compromised immune systems.

#### Example Data – Enterococci or Coliphage

If the system had sampled for (and found) enterococci or coliphage as their fecal indicator, the table would read as shown in the sample below.

Example CCR Table Excerpt – Enterococci or Coliphage

| Contaminant | MCL | MCLG | Your Water | Range | Sample Date | Violation | Typical Source |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Enterococci (at the groundwater source)1 | TT | N/A | 2 positive samples | ND - 1 | 2026 | No | Human or animal fecal waste |
| Coliphage (at the groundwater source)1 | TT | N/A | 2 positive samples | ND - 1 | 2026 | No | Human or animal fecal waste |

1 Special notice required text and health effects language would be provided in the CCR – possibly in a footnote to the table as shown in the previous example for E. coli.

Health Effects: Fecal indicators are microbes whose presence indicates that the water may be contaminated with human or animal wastes. Microbes in these wastes can cause short-term effects, such as diarrhea, cramps, nausea, headaches, or other symptoms. They may pose a special health risk for infants, young children, some of the elderly, and people with severely-compromised immune systems.

### Example: Reporting of GWR treatment technique violations for failure to take corrective action for fecal indicator-positive groundwater source sample

If in the example shown on the previous page, the system did not complete corrective action(s) within 120 days (or earlier if required by State Water Board) of the fecal indicator-positive source sample, or failed to be in compliance with a State Water Board-approved corrective action plan and schedule, it will be in violation of the TT.

Example CCR Table Excerpt

| TT Violation | Explanation | Length | Steps Taken to Correct the Violation | Health Effect Language |
| --- | --- | --- | --- | --- |
| Corrective Action for Groundwater Fecal Indicator Source Sample(s) | We were required to take corrective action to address the fecal contamination in our well. | 3 months | We have contacted the State Water Resources Control Board and are now on a corrective action plan. We will abandon the contaminated well and drill a new one. We will have the new well completed by [date], and the old well will be abandoned by [date]. | Inadequately protected or treated water may contain disease-causing organisms. These organisms can cause symptoms such as diarrhea, nausea, cramps, and associated headaches. |

The table below shows how to determine the “sample result” for an initial sample with 1 or 2 confirmation samples (compliance: initial or averaged with confirmation samples).

| Xylenes (µg/L) | Initial Sample | Confirmation 1 | Confirmation 2 | Sample Result |
| --- | --- | --- | --- | --- |
| Well 1 | 36 | ND | 16 | 26 |
| Well 2 | 18 | 6 | not needed | 12 |
| Well 3 | 7 | ND | ND | ND |

## APPENDIX G: CCR Certification Form (Suggested Format)

Consumer Confidence Report

Certification Form

(to be submitted with a copy of the CCR)

(To certify electronic delivery of the CCR, submit to your district office or LPA, depending on who oversees your regulatory compliance)

| Water System Name: | [INSERT WATER SYSTEM NAME] |
| --- | --- |
| Water System Number: | [ENTER WATER SYSTEM NUMBER] |

The water system named above hereby certifies that its Consumer Confidence Report was distributed on [INSERT DATE] to customers (and appropriate notices of availability have been given). Further, the system certifies that the information contained in the report is correct and consistent with the compliance monitoring data previously submitted to the State Water Resources Control Board, Division of Drinking Water.

| Certified by: | [INSERT NAME] |
| --- | --- |
| Name: | [INSERT NAME] |
| Signature: | [INSERT SIGNATURE] |
| Title: | [INSERT TITLE] |
| Phone Number: | [INSERT PHONE NUMBER] |
| Date: | [INSERT DATE] |

To summarize report delivery used and good-faith efforts taken, please complete the below by checking all items that apply and fill-in where appropriate:

- CCR was directly delivered by at least one of the following methods:

- Mailing or hand delivering a copy of the CCR

- Mailing a notification that the report is available on a website via a direct link

- Emailing a direct link or electronic version of the report

- Other: [INSERT DELIVERY METHODS]

- “Good faith” efforts were used to reach non-bill paying consumers. Those efforts included the following methods:

- Posting the CCR on the internet at [INSERT INTERNET ADDRESS]

- Mailing the CCR or postcards with links to all service addresses and/or postal patrons within the service area (attach zip codes used)

- Using an opt in notification system to send emails and/or texts with links to the CCR to interested consumers

- Advertising the availability of the CCR in news media (attach copy of press release) and/or social media (attach copy of post)

- Publication of the CCR in a local newspaper or newsletter (attach a copy of the published notice, including name of newspaper and date published)

- Posting the CCR or notice of availability with links (or equivalent, such as QE codes) in public places (attach a list of locations)

- Delivering multiple copies of CCR to single-billed addresses serving several persons, such as apartments, businesses, and schools

- Delivering the CCR to community organizations (attach a list of organizations)

- Holding a public meeting to educate consumers on the CCR

- Other (attach a list of other methods used)

- For systems serving at least 50,000 persons: Posted CCR on a publicly-accessible internet site at the following address: [INSERT INTERNET ADDRESS]

- For investor-owned utilities: Delivered the CCR to the California Public Utilities Commission

This form is provided as a convenience for use to meet the certification requirement of the California Code of Regulations, section 64483(c).

## APPENDIX H: Translations of “Note of Importance” for CCR

Pursuant to the California Code of Regulations, section 64481(l), your CCR is required to contain information in Spanish on the importance of the report or contain a telephone number or address where Spanish-speaking residents may contact the water system to obtain a translated copy of the report or assistance in Spanish. For any language spoken by a non-English speaking group that exceeds 1,000 residents or 10 percent of the residents in a community, the CCR is required to contain the same information in the appropriate language(s).

In addition to the English CCR template, the State Water Board is offering CCR templates in Spanish, Chinese (Simplified), and Hmong, for your convenience. These templates are available at https://www.waterboards.ca.gov/drinking_water/certlic/drinkingwater/CCR.html. The State Water Board is also offering translations of the below statement in 18 of the languages most spoken in California. If a utility has a translation not available on this website that it would like to share with other utilities, please email DDWRegUnit@waterboards.ca.gov.

This report contains important information about your drinking water. Please contact [Insert Water System’s Name] at [Insert Water System’s Address and/or Phone Number] for assistance in [Language].

Arabic

يحتوي هذا التقرير على معلومات مهمة عن مياه الشرب. يُرجى التواصل مع [Insert Water System’s Name] عبر [Insert Water System’s Address and/or Phone Number] للحصول على مساعدة باللغة العربية.

Armenian

Այս զեկույցը պարունակում է կարեւոր տեղեկություններ ձեր խմելու ջրի մասին: Խնդրում ենք դիմել [Insert Water System’s Name] ջրի համակարգի հասցեով [Insert Water System’s Address] կամ հեռախոսահամարով [Insert Water System’s Phone Number] հայերենով օգնություն ստանալ համար:

Cantonese

本報告包含閣下飮用水嘅重要訊息。 如需廣東話垂詢，請聯絡 [Insert Water System’s Name, with Address and/or Phone Number]。

Farsi, Persian

این گزارش حاوی اطلاعات مهمی در مورد آب آشامیدنی شماست.

لتفا بر‌ای کسب اطلاعات به سازمان آب آشامیدنی [Insert Water System’s Name] که در آدرس [Insert Water System’s Address] تماس بگیرید. شماره تلفن [Insert Water System’s Phone Number] است.

French

Ce rapport contient des informations importantes concernant votre eau potable. Veuillez contacter [Insert Water System’s Name] à [Insert Water System’s Address and/or Phone Number] pour de plus amples informations en français.

German

Dieser Bericht enthält wichtige Information über Ihr Trinkwasser. Bitte wenden Sie sich an [Insert Water System’s Name] unter [Insert Water System’s Address and/or Phone Number], um Unterstützung in deutscher Sprache zu erhalten.

Hindi

इस रिपोर्ट में आपके पीने के जल से सम्बंधित महत्वपूर्ण जानकारी है l हिंदी में सहायता के लिए, [Insert Water System’s Name] को [Insert Water System’s Address] अथवा [Insert Water System’s Phone Number] पर संपर्क करें l

Hmong

Tsab ntawv no muaj cov ntsiab lus tseem ceeb hais txog koj cov dej haus. Thov hu rau [Insert Water System’s Name] ntawm [Insert Water System’s Address and/or Phone Number] yog koj xav tau kev pab hais lus Hmoob.

Japanese

この報告書には上水道に関する重要な情報が記されております。
ご質問等ございましたら、[Insert Water System’s Name, with Address and/or Phone Number]まで日本語でご連絡下さい。

Korean

이 보고서는 당신의 식수에 관한 중요한 정보를 포함하고 있습니다. 한국어로 된 도움을 원하시면 [Insert Water System’s Name, with Address and/or Phone Number] 로 문의 하시기 바랍니다.

Mandarin (Simplified)

这份报告含有关于您的饮用水的重要讯息。请用以下地址和电话联系 [Insert Water System’s Name] 以获得中文的帮助: [Insert Water System’s Address and/or Phone Number]

Mandarin (Traditional)

這份報告含有關於您的飲用水的重要訊息。請用以下地址和電話聯繫 [Insert Water System’s Name]以獲得中文的幫助: [Insert Water System’s Address and/or Phone Number]

Portuguese

Este relatório contém informação importante sobre sua água potável. Por favor entre em contato com [Insert Water System’s Name] a [Insert Water System’s Address and/or Phone Number] para auxílio em portugués.

Punjabi

ਐੱਸ ਰਿਪੋਟ ਵਿਚ ਤੁਵਾੜੇ ਪੀਣੇ ਦੇ ਵਾਰੇ ਮਹੱਤਵਪੂਰਨ ਸੂਚਨਾ ਹੈ l ਪੰਜਾਬੀ ਵਿਚ ਮਦਦ ਲਈ, [Insert Water System’s Name] ਨੂੰ [Insert Water System’s Address] ਜਾਂ [Insert Water System’s Phone Number] ਤੇ ਸੰਪਰਕ ਕਰੋ |

Russian

Этот отчет содержит важную информацию о вашей питьевой воде. Пожалуйста, свяжитесь с [Insert Water System’s Name] по [Insert Water System’s Address and/or Phone Number] для получения помощи на русском языке.

Spanish

Este informe contiene información muy importante sobre su agua para beber. Favor de comunicarse [Insert Water System’s Name] a [Insert Water System’s Address and/or Phone Number] para asistirlo en español.

Tagalog

Ang pag-uulat na ito ay naglalaman ng mahalagang impormasyon tungkol sa inyong inuming tubig. Mangyaring makipag-ugnayan sa [Insert Water System’s Name and Address] o tumawag sa [Insert Water System’s Phone Number] para matulungan sa wikang Tagalog.

Thai

รายงานฉบับนี้มีข้อมูลที่สำคัญเกี่ยวกับน้ำประปาของท่าน
กรุณาติดต่อ [Insert Water System’s Name] ที่ [Insert Water System’s Address and/or Phone Number] เพื่อการช่วยเหลือในภาษาไทย

Vietnamese

Báo cáo này chứa thông tin quan trọng về nước uống của bạn. Xin vui lòng liên hệ [Insert Water System’s Name] tại [Insert Water System’s Address and/or Phone Number] để được trợ giúp bằng tiếng

## APPENDIX I: Source Water Protection and Water Conservation Tips for Consumers

Examples of tips for source water protection and water conservations that could appear in a CCR are shown in the tables below.

Examples of Source Water Protection Tips for Consumers

Protection of drinking water is everyone’s responsibility. You can help protect your community’s drinking water source in several ways: - Eliminate excess use of lawn and garden fertilizers and pesticides – they contain hazardous chemicals that can reach your drinking water source. - Pick up after your pets. - If you have your own septic system, properly maintain your system to reduce leaching to water sources or consider connecting to a public water system. - Dispose of chemicals properly; take used motor oil to a recycling center. - Volunteer in your community. Find a watershed or wellhead protection organization in your community and volunteer to help. If there are no active groups, consider starting one. Use U.S. EPA’s Adopt Your Watershed to locate groups in your community, or visit the Watershed Information Network’s How to Start a Watershed Team. - Organize a storm drain stenciling project with your local government or water system. Stencil a message next to the street drain reminding people “Dump No Waste – Drains to River” or “Protect Your Water”. Produce and distribute a flyer for households to remind residents that storm drains dump directly into your local water body.

Water Conservation Tips for Consumers – Example Language

Did you know that the average U.S. household uses approximately 400 gallons of water per day or 100 gallons per person per day? Luckily, there are many low-cost and no-cost ways to conserve water. Small changes can make a big difference – try one today and soon it will become second nature. - Take short showers – a 5 minutes shower uses 4 to 5 gallons of water compared to up to 50 gallons for a bath. - Shut off water while brushing your teeth, washing your hair, and shaving and save up to 500 gallons a month. - Use a water-efficient showerhead. They are inexpensive, easy to install, and can save you up to 750 gallons a month. - Run your clothes washer and dishwasher only when they are full. You can save up to 1,000 gallons a month. - Water plants only when necessary. - Fix leaking toilets and faucets. Faucet washers are inexpensive and take only a few minutes to replace. To check your toilet for a leak, place a few drops of food coloring in the tank and wait. If it seeps into the toilet bowl without flushing, you have a leak. Fixing it or replacing it with a new, more efficient model can save up to 1,000 gallons a month. - Adjust sprinklers so only your lawn is watered. Apply water only as fast as the soil can absorb it and during the cooler parts of the day to reduce evaporation. - Teach your kids about water conservation to ensure a future generation that uses water wisely. Make it a family effort to reduce next month’s water bill! - Visit https://www.epa.gov/watersense for more information.

## APPENDIX J: Acronyms, Abbreviations, and Initials

| AL | Action Level |
| --- | --- |
| AWQR | Annual Water Quality Report |
| CCR | Consumer Confidence Report |
| CDC | Centers for Disease Control |
| CFR | Code of Federal Regulations |
| CT | Contact-Time |
| CWS | Community Water System |
| DDW | Division of Drinking Water |
| DLR | Detection Limit for Purposes of Reporting |
| DWSRF | Drinking Water State Revolving Fund |
| FBRR | Filter Backwash Recycling Rule |
| GWR | Ground Water Rule |
| GWUDI | Groundwater Under the Direct Influence of Surface Water |
| HAA5 | Sum of Five Regulated HAAs, i.e., Monochloroacetic Acid, Monobromoacetic Acid, Dichloroacetic Acid, Dibromoacetic Acid, and Trichloroacetic Acid |
| HAA6Br | Sum of Bromochloroacetic Acid, Bromodichloroacetic Acid, Dibromoacetic Acid, Dibromochloroacetic Acid, Monobromoacetic Acid, and Tribromoacetic Acid |
| HAA9 | Sum of Bromochloroacetic Acid, Bromodichloroacetic Acid, Chlorodibromoacetic Acid, Dibromoacetic Acid, Dichloroacetic Acid, Monobromoacetic Acid, Monochloroacetic Acid, Tribromoacetic Acid, and Trichloroacetic Acid |
| HIV/AIDS | Human Immunodeficiency Virus / Acquired Immunodeficiency Syndrome |
| HMX | Octahydro-1,3,5,7-tetranitro-1,3,5,7-tetrazocine |
| HSC | Health and Safety Code |
| IESWTR | Interim Enhanced Surface Water Treatment Rule |
| LCR | Lead and Copper Rule |
| LPA | Local Primacy Agency |
| LRAA | Locational Running Annual Average |
| LT1ESWTR | Long-Term 1 Enhanced Surface Water Treatment Rule |
| LT2ESWTR | Long-Term 2 Enhanced Surface Water Treatment Rule |
| MCL | Maximum Contaminant Level |
| MCLG | Maximum Contaminant Level Goal |
| MDA | Minimum Detectable Activity |
| MFL | Million fibers per liter |
| mg/L | milligrams per liter |
| MIBK | Methyl Isobutyl Ketone |
| MRDL | Maximum Residual Disinfectant Level |
| MRDLG | Maximum Residual Disinfectant Level Goal |
| mrem | millirems |
| mrem/yr | millirems per year |
| N/A | Not Applicable |
| ND | Non-Detected |
| NDEA | N-Nitrosodiethylamine |
| NDMA | N-Nitrosodimethylamine |
| NDPA | N-Nitrosodi-n-propylamine |
| NL | Notification Level |
| NTNCWS | Nontransient-noncommunity Water System |
| NTU | Nephelometric Turbidity Units |
| pCi/L | picocuries per liter |
| PDWS | Primary Drinking Water Standard |
| PFBS | Perfluorobutanesulfonic acid |
| PFHpA | Perfluoroheptanoic acid |
| PFHxS | Perfluorohexanesulfonic acid |
| PFNA | Perfluorononanoic acid |
| PFOA | Perfluorooctanoic Acid |
| PFOS | Perfluorooctanesulfonic Acid |
| PHG | Public Health Goal |
| ppb | parts per billion |
| ppm | parts per million |
| ppq | parts per quadrillion |
| ppt | parts per trillion |
| PWS | Public Water System |
| RAA | Running Annual Average |
| RDX | Hexahydro-1,3,5-trinitro-1-3-5-triazine |
| RTCR | Revised Total Coliform Rule |
| SDWA | Safe Drinking Water Act |
| Stage 1 D/DBPR | Stage 1 Disinfectants and Disinfection Byproducts Rule |
| Stage 2 D/DBPR | Stage 2 Disinfectants and Disinfection Byproducts Rule |
| State Water Board | State Water Resources Control Board |
| SWTR | Surface Water Treatment Rule |
| TBA | Tertiary Butyl Alcohol |
| TCP | 1,2,3-Trichloropropane |
| TCR | Total Coliform Rule |
| TNT | 2,4,6-Trinitrotoluene |
| TOC | Total Organic Carbon |
| TT | Treatment Technique |
| TTHM | Total Trihalomethanes, or Sum of Four Regulated THMs, i.e., Chloroform, Bromodichloromethane, Dibromochloromethane, and Bromoform |
| UCMR | Unregulated Contaminant Monitoring Rule |
| U.S. EPA | United States Environmental Protection Agency |

From the Demographic Statistical Atlas of the United States, December 19, 2024, https://statisticalatlas.com/state/California/Languages

U.S. EPA is essentially silent on the issue of reporting federal UCMR contaminants beyond the previous calendar year’s detections, other than to say it is not required and that data older than five years need not be reported. As a result, the State Water Board recommends systems to report data for five years from the date of the last sampling.

If reporting results for Ra-226 and Ra-228 as individual constituents, the PHG is 0.05 pCi/L for Ra-226 and 0.019 pCi/L for Ra-228.

From the Demographic Statistical Atlas of the United States, December 19, 2024, https://statisticalatlas.com/state/California/Languages